LAKES OIL N.L.

A.C.N. (004 247 214)

Banjo No.1A		PEP 155 (east)

DAILY GEOLOGICAL REPORT No. 8	Date: 16 Jan 06

Depth: 229.6m		Progress: 29.6m		Days from Spud: 06

Rig:			Drilltec		GL(ASL):		80m

Drilling Rep:	Tim O’Brien	RT (AGL):		0.8m

Geologist:		Ben Edwards	Last Casing:	6.0” at 86m

Comments: Pull casing from 199.5m, drill 4” hole to 229.6m

| Interval (mRT) | Hydrocarbon Show Summary | Gas |
| --- | --- | --- |
| 200-229.6 | No oil fluorescence | Nil |

| Formation Tops: | Prognosed (mRT) | Actual٭ (mRT) | Actual٭ (mSS) | Difference٭ (High/Low) |
| --- | --- | --- | --- | --- |
| Pleistocence Dunes | Absent | 0.8 | +80 | - |
| Haunted Hills Gravel | 0.8 | 057 | +22.2 | 56.2 Low |
| Jemmy’s Point Formation | 15 | 066 | +13.2 | 41 Low |
| Gippsland Limestone | 50 | 082 | -1.2 | 32 Low |
| Lakes Entrance Formation: Seacombe Marl Member | 190 | 133 | -52.2 | 57 High |
| Lakes Entrance Formation: Cunninghame Greensand | Absent | 222 | -161.2 | - |
| Colquhoun Gravels | 360 | 225 | -164.2 | 135 High |
| Granite Basement | 400 |  |  |  |
| Total Depth | 600 |  |  |  |

٭Provisional, based on mudlog

LAKES OIL N.L.

A.C.N. (004 247 214)

Banjo No.1A		PEP 155 (east)

DAILY GEOLOGICAL REPORT No. 8

Lithological and Fluorescence Description

| Interval (m) | Description |
| --- | --- |
| 198-207 | LAKES ENTRANCE FORMATION: SEACOMBE MARL MEMBER: Marlstone (80%) interbedded with Sandstone (20%) MARLSTONE: very light bluish grey-medium bluish grey, fine-very fine, silty, trace quartz, common fossiliferous fragments; bryozoa gastropod echinoid spines shells, soft, dispersive, moderately calcareous, sub-fissile. SANDSTONE: white-very light yellow, subrounded-well rounded, well sorted, fine, dominant translucent-opaque subrounded-well rounded quartz, trace dark green-black very fine well rounded glauconitic nodules, rare pyrite, pale yellow-light yellowish brown silty matrix, trace weak calcite cement, fair inferred porosity, no oil fluorescence. |
| 207-222 | LAKES ENTRANCE FORMATION: SEACOMBE MARL MEMBER: Marlstone (90%) interbedded with Siltstone (10%) MARLSTONE: very light bluish grey-medium bluish grey, fine-very fine, slightly silty, rare limestone lithics with strong calcite cement, rare pyrite, trace quartz, common fossiliferous fragments; bryozoa gastropod echinoid spines shells, soft, dispersive, moderately calcareous, sub-fissile. SILTSTONE: medium brownish grey-medium brown, very fine-fine, moderately sorted, moderately arenaceous, rare translucent-opaque very fine-fine quartz, trace very fine black lithics, trace pyrite, poor inferred porosity, no oil fluorescence. |
| 222-225 | LAKES ENTRANCE FORMATION: CUNNINGHAME GREENSAND: Greensand: (100%) GREENSAND: medium green-dark green, medium-coarse dominantly medium, subrounded-well rounded dominantly well rounded, common rounded-well rounded translucent-opaque medium-coarse quartz with occasional iron staining, abundant dark green-black rounded-well rounded medium-coarse glauconite nodules, trace fossiliferous material mainly bryozoa, trace pyrite, rare off white-very light grey claystone matrix, weak calcite cement, fair-good inferred porosity, dull yellow mineral fluorescence, no oil fluorescence. |
| 225-229.6 | COLQOHOUN GRAVELS: Greensand (10%) grading into Calcareous Sandstone (90%) GREENSAND: medium green-dark green, medium-coarse dominantly medium, subrounded-well rounded dominantly well rounded, common rounded-well rounded translucent-opaque medium-coarse quartz with occasional iron staining, abundant dark green-black rounded-well rounded medium-coarse glauconite nodules, trace fossiliferous material mainly bryozoa, trace pyrite, rare off white-very light grey claystone matrix, weak calcite cement, fair-good inferred porosity, dull yellow mineral fluorescence, no oil fluorescence. CALCAREOUS SANDSTONE: off white-very light yellow, fine-medium, well sorted, subrounded-rounded dominantly rounded, abundant rounded-well rounded fine translucent-opaque quartz, common dark green-black glauconite, common fossiliferous material mainly bryozoa, common white claystone matrix, trace pyrite, moderate calcite cement, fair visual porosity, bright yellow mineral fluorescence, no oil fluorescence. |

LAKES OIL N.L.				CUTTINGS DESCRIPTIONS

WELL NAME: 		Banjo No.1A		DATE 16 JAN 06

GEOLOGIST:		Ben Edwards		PAGE:	1

| Interval (m) | % | Description |
| --- | --- | --- |
| 198-201 |  | Sample dominated by cement |
| 201-204 | 70 | MARLSTONE: very light bluish grey-medium bluish grey, fine-very fine, silty, trace quartz, common fossiliferous fragments; bryozoa gastropod echinoid spines shells, soft, dispersive, moderately calcareous, sub-fissile. |
|  | 30 | SANDSTONE: white-very light yellow, subrounded-well rounded, well sorted, fine, dominant translucent-opaque subrounded-well rounded quartz, trace dark green-black very fine well rounded glauconitic nodules, rare pyrite, pale yellow-light yellowish brown silty matrix, trace weak calcite cement, fair inferred porosity, no oil fluorescence. |
| 204-207 | 90 | MARLSTONE: as above |
|  | 10 | SANDSTONE: as above |
| 207-210 | 80 | MARLSTONE: as above |
|  | 20 | SILTSTONE: medium brownish grey-medium brown, very fine-fine, moderately sorted, moderately arenaceous, common translucent-opaque very fine-fine quartz, trace very fine black lithics, poor inferred porosity, no oil fluorescence. |
| 210-216 | 90 | MARLSTONE: very light bluish grey-medium bluish grey, fine-very fine, slightly silty, rare limestone lithics with strong calcite cement, rare pyrite, trace quartz, common fossiliferous fragments; bryozoa gastropod echinoid spines shells, soft, dispersive, moderately calcareous, sub-fissile. |
|  | 10 | SILTSTONE: as above |
| 216-219 | 100 | MARLSTONE: as above |
| 219-222 | 80 | MARLSTONE: as above |
|  | 20 | SILTSTONE: medium brownish grey-medium brown, very fine-fine, moderately sorted, moderately arenaceous, rare translucent-opaque very fine-fine quartz, trace very fine black lithics, trace pyrite, poor inferred porosity, no oil fluorescence. |
| 222-225 | 100 | GREENSAND: medium green-dark green, medium-coarse dominantly medium, subrounded-well rounded dominantly well rounded, common rounded-well rounded translucent-opaque medium-coarse quartz with occasional iron staining, abundant dark green-black rounded-well rounded medium-coarse glauconite nodules, trace fossiliferous material mainly bryozoa, trace pyrite, rare off white-very light grey claystone matrix, weak calcite cement, fair-good inferred porosity, dull yellow mineral fluorescence, no oil fluorescence. |
| 225-229.6 | 10 | GREENSAND: as above |
|  | 90 | CALCAREOUS SANDSTONE: off white-very light yellow, fine-medium, well sorted, subrounded-rounded dominantly rounded, abundant rounded-well rounded fine translucent-opaque quartz, common dark green-black glauconite, common fossiliferous material mainly bryozoa, common white claystone matrix, trace pyrite, moderate calcite cement, fair visual porosity, bright yellow mineral fluorescence, no oil fluorescence. |