LAKES OIL N.L. A.C.N. (004 247 214) BOUNDARY CREEK No.2 PEP 157

DAILY GEOLOGICAL REPORT No. 31 Date: 9-11-2005

Depth: 2041m Progress: 67m Days from Spud: 31

Rig: HUNT RIG No.2 GL(AHD): 74.0m Drilling Rep: Lou DeVattimo RT: (datum) 77.65m Geologist: David Horner Last Casing: 178mm at 1097m

0600 hrs Update: Drill ahead with 156mm hole at 2065m. Lithology Claystone (80%) with laminated very fine Sandstone (20%), background gas 0.5 units (C1 100%).

Comments: RIH with new bit, drill ahead with 156mm hole to 2041m. Trip gas 3.5 units.

| Interval (mRT) | Hydrocarbon Show Summary | Gas |
| --- | --- | --- |
| 1974-2041 | Strzelecki Formation - No Show Maximum gas 2.5 units from coal at 2032m. | TG tr-2.5u C1 100% |

| Formation Tops: | Prognosed (mRT) | Actual* (mRT) | Actual* (mSS) | Difference* (High/Low) |
| --- | --- | --- | --- | --- |
| LaTrobe Group Strzelecki Formation T.D. | 4.7 197 2500 | 4.7 227 | +73 -149 | 0 30 Low |

*Provisional, based on mudlog

LAKES OIL N.L. A.C.N. (004 247 214) BOUNDARY CREEK No.2 PEP 157

DAILY GEOLOGICAL REPORT No. 31

| Lithological and Fluorescence Description |  |
| --- | --- |
| Interval (m) | Description |
| 1974-2035 | STRZELECKI FORMATION Sandstone (70%) interbedded and laminated with Claystone (30%) with minor Coal (Trace). SANDSTONE: light to medium green grey, very fine to occasionally medium, dominantly fine, subangular to subrounded, moderately sorted, strong silica and weak to moderate calcareous cements, abundant white argillaceous matrix - matrix supported, abundant altered feldspar, common grey green lithics, rare orange brown and black lithics, trace quartz grains, trace black coal detritus, hard, no visual porosity, no oil fluorescence. CLAYSTONE: light to medium dark grey to medium brown grey, very silty in part - grades to siltstone, very finely arenaceous with altered feldspars grains in part, slightly calcareous where arenaceous, trace black carbonaceous flecks, trace black coal detritus, common micromica, moderately hard, subfissile. COAL: black to dark brown grey, very argillaceous in part, earthy with blocky fracture where argillaceous, subvitreous with subconchoidal fracture where clean, hard, brittle. CUT: The coal has no fluorescence but gives a very weak dull pale yellow crush cut. |
| 2035-2041 | STRZELECKI FORMATION Claystone (80%) with laminated Sandstone (20%). SANDSTONE: light to medium green grey, very fine to occasionally fine, dominantly very fine, subangular to subrounded, moderately sorted, strong silica and weak calcareous cements, abundant white argillaceous matrix - matrix supported, abundant altered feldspar, common grey green lithics, rare orange brown and black lithics, trace quartz grains, trace black coal detritus, hard, no visual porosity, no oil fluorescence. CLAYSTONE: off white to medium dark grey to medium brown grey, rarely dark grey, very silty in part, very finely arenaceous with altered feldspars grains in part, slightly calcareous where arenaceous, trace black carbonaceous flecks, trace black coal detritus, common micromica, common quartz and calcite veining in part, moderately hard, subfissile. |