|  | WELL |  | BMA A6A |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DATE |  | 12-02-2006 |  |  |  |  |  |
|  | DAY # |  | 13 |  |  |  |  |  |
| MIDNIGHT ACTIVITY |  | Wiper trip to shoe. |  |  | ROP DATA (LAST 24 hours) |  | m/hr | DEPTH (m) |
| MIDNIGHT DEPTH |  | 3256.0 mMDRT |  |  | MINIMUM |  | 0.9 | 3218.0 |
| TVD |  | 1994.7 mTVDRT |  |  | MAXIMUM |  | 37.0 | 3234.5 |
| PROGRESS LAST 24 hrs |  | 41.0 |  |  | AVERAGE |  | 5.9 |  |

|  | TOP (mMDRT) | TOP (mTVDRT) | TOP (mTVDSS) RTtoMSL (32.82m) | HIGH/ LOW (mTVD) | PROGNOSED TOP (mMDRT) | PROGNOSED TOP (mTVDRT) |
| --- | --- | --- | --- | --- | --- | --- |
| Kick Off Point | 855.0 | 807.96 | -775.1 |  | 855.0 | 808.0 |
| Lakes Entrance | 961.5 | 908.3 | -875.5 | 1.5 LOW | 960.8 | 906.8 |
| Top of Latrobe | 2779.5 | 1792.3 | -1759.5 | 14.0 LOW | 2735.9 | 1778.3 |
| TCC (N-1 Reservoir) | 3090.0 | 1920.8 | -1888.0 | 1.3 HIGH | 3086.5 | 1922.1 |
| Base of Waste Rock | 3098.5 | 1924.5 | -1891.7 | 4.4 HIGH | 3103.0 | 1928.9 |
| Current OWC |  |  |  |  | 3141.7 | 1944.8 |
| CbF2 horizon | 3138.0 | 1941.7 | -1908.9 | 1.3 HIGH | 3136.1 | 1943.0 |
| CbF1 | 3144.0 | 1944.4 | -1911.6 | 3.0 HIGH | 3146.9 | 1947.4 |
| OOWC | 3183.6 | 1961.8 | -1929.0 | 0.0 | 3183.2 | 1961.8 |
| CBSB | 3152.5 | 1948.1 | -1925.3 | 4.3 HIGH | 3160.3 | 1952.4 |
| PKSB | 3216.0 | 1976.3 | -1943.5 | 3.3 LOW | 3210.5 | 1973.0 |
| MVSB |  |  |  |  | 3247.0 | 1988.8 |
| Total Depth | 3256.0 | 1994.7 | -1961.9 |  | 3256.4 | 1991.8 |

| ##### Current Formation | LaTrobe: Sandstone, Claystone, Siltstone, Coal. |
| --- | --- |

| GAS DATA ## FCP/FGP | DEPTH (m) | TOTAL GAS (U) | C1 % | C2 % | ## C3 % | iC4 % | nC4 % | C5 % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MAXIMUM | 3243.0 | 26 | 0.205 | 0.036 | 0.015 | 0.001 | 0.003 | 0.003 |
|  | 3243.0 | 66 | 0.805 | 0.101 | 0.057 | 0.094 | 0.017 | 0.014 |
| BACKGROUND |  | 8 |  |  |  |  |  |  |
|  |  | 20 |  |  |  |  |  |  |
| TRIP GAS |  |  |  |  |  |  |  |  |

| BIT DATA |  | PRESENT BIT | LAST BIT |
| --- | --- | --- | --- |
| BIT No. |  | 1 |  |
| SIZE (in) |  | 8½ |  |
| MANUFACTURER/TYPE |  | Smith S73PX |  |
| JETS (/32 inch) |  | 6 x 20 TFA 1.841 |  |
| DEPTH IN (m) |  | 851.0 |  |
| DEPTH OUT (m) |  | IN |  |
| METRES ON RUN (m) |  | 2705.0 |  |
| HRS ON BTM 24hrs/total |  | 6.9 / 82.6 |  |
| BIT CONDITION |  |  |  |
| RPM RANGE LAST 24 hrs |  | 75-110 TDRPM / 170-175 MMRPM |  |
| Pump Hrs 24 hrs / total |  | 15.5 / 198.6 |  |
| Krevs 24 hrs / total |  | 116 / 1240 |  |
| 24 HOUR SUMMARY | Drill from 3215.0 mMDRT to 3230.0 mMDRT. Rotate and circulate while replacing swab on mud pump #1. Drill from 3230.0 mMDRT to 3256.0 mMDRT. Rotate and circulate, pump 20 bbl Hi Vis sweep. Rack back 1 stand per ½ hour from 3256.0 mMDRT to 3198.0 mMDRT. Slug pipe and pull out hole to tight spot at 1548.0 mMDRT. Run in hole to 1618.0 mMDRT and circulate hole clean. Pull out of hole to tight spot at 1548.0 mMDRT. Back ream tight hole from 1548.0 mMDRT to 1526.0 mMDRT, packing off. Run in hole to 1610.0 mMDRT and circulate hole clean. Pull out of hole to tight spot at 1497.0 mMDRT. Wash and ream tight hole. Pull out of hole to tight spot at 1350.0 mMDRT. Wash and ream tight hole. Pull out of hole to tight spot at 1272.0 mMDRT. Wash and ream tight hole. Pull out of hole to tight spot at 1065.0 mMDRT. Circulate and work string to hole clean. . Pull out of hole to tight spot at 1025.0 mMDRT. Back ream and work tight hole. |  |  |
| 05:00 AM UPDATE 13-02-2006 ### DEPTH @ 05:00 hours | Run in hole to 1647.0 mMDRT. 3256.0 mMDRT (1994.67 mTVDRT) |  |  |