LAKES OIL N.L

DAILY DRILLING REPORT

| WELL : | Wombat-4 | REPORT No. : | 29 | DFS: | 29 | DATE : | 14-11-09 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2400 DEPTH : | 2,500 | LAST DEPTH : | 2,500 | 24 HR PROGRESS: |  | PTD : | Total Depth |
| TOTAL FUEL : | 2,300 | FUEL USAGE : | 1,200 | DAYCOST : $ |  | CUM. COST : $ |  |
| 0600 OPS : | Recovering Schlumberger core samples |  |  |  |  |  |  |
| Next 24 Hrs Op | Run DST |  |  |  |  |  |  |
| Site Personnel: | Lou DeVattimo - Supervisor |  |  |  |  |  |  |

|  |  | BIT INFORMATION |  |  |  |  |  |  |  | # OPS BREAKDOWN |  |  |  |  |  |  |  |  |  | # MUD PROPERTIES @ 22.30 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Bit Number |  |  | 5 Re-Run#4 |  | 6 New |  |  | Drilling |  |  |  |  |  |  |  |  |  | Density |  |  | 1.297 |  |  | S.G. |  |  |
|  |  | Size (mm) |  |  | 156 |  | 156 |  |  | Tripping |  |  |  |  | 8.5 |  |  |  |  | Viscosity |  |  | 42 |  |  | Sec |  |  |
|  |  | Make |  |  | Smith |  | Reed |  |  | Downtime ( Float ) |  |  |  |  |  |  |  |  |  | Water Loss |  |  | 14.0 |  |  | Cc |  |  |
|  |  | Type |  |  | MDi613 |  | R22AP |  |  | Wireline logging |  |  |  |  | 15.5 |  |  |  |  | pH |  |  | 8.0 |  |  | Strip |  |  |
|  |  | IADC Code |  |  |  |  | 5-1-7 |  |  | Testing/tools |  |  |  |  |  |  |  |  |  | Filter Cake |  |  | 4 |  |  | 32 nd |  |  |
|  |  | Serial Number |  |  | JD1042 |  | KD1019 |  |  | Circulate Sample |  |  |  |  |  |  |  |  |  | PV / YP |  |  | 16/19 |  |  | Cp/lb/100’ |  |  |
|  |  | Nozzles (32) |  |  | 3 x 12 |  | 3 x 14 |  |  | Casing |  |  |  |  |  |  |  |  |  | Gels |  |  | 5 / 16 |  |  | S/m |  |  |
|  |  | Depth in (m) |  |  | 1378 / 1478 |  | 2403 |  |  | Cementing |  |  |  |  |  |  |  |  |  | Sand |  |  | 0.50 |  |  | % Vol |  |  |
|  |  | Depth out (m) |  |  | / 2403 |  | 2500 |  |  | Surveys |  |  |  |  |  |  |  |  |  | Solids |  |  | 14.6 |  |  | % Vol |  |  |
|  |  | Total Metres |  |  | 1025 |  | 97 |  |  | Circulate & Condition |  |  |  |  |  |  |  |  |  | Oil |  |  |  |  |  | % Vol |  |  |
|  |  | Hours |  |  | 123 |  | 38.5 |  |  | Rig Service/slip & cut |  |  |  |  |  |  |  |  |  | K+ |  |  | 36,750 |  |  | Mg/l |  |  |
|  |  | WOB (kg) |  |  | 2,000/5000 |  | 10-18k |  |  | Rig up/ d own bop |  |  |  |  |  |  |  |  |  | Chlorides |  |  | 45,000 |  |  | Mg/l |  |  |
|  |  | RPM |  |  | 90 /100 |  | 90 |  |  | Drill out Cmt |  |  |  |  |  |  |  |  |  | PHPA |  |  |  |  |  |  |  |  |
|  |  | Condition |  |  | 1-2-ER-A-X-2mm-NO-HRS |  | 3-3-ER-A-E-1-TD |  |  | Reaming |  |  |  |  |  |  |  |  |  | KCL |  |  | 7.0 |  |  | % |  |  |
|  |  | Impact Force |  |  | 237 |  | 176 |  |  | Wait On Cement |  |  |  |  |  |  |  |  |  | Daily Cost $ |  |  | 7,283 |  |  |  |  |  |
|  |  | Jet Velocity |  |  | 215 |  | 158 |  |  | Test BOPs |  |  |  |  |  |  |  |  |  | Cum Cost $ |  |  | 82,281 |  |  |  |  |  |
|  |  | B.H.A.: 5 |  | 156mm Bit-NBS-NMDC-1 x DC-STB-13 x DC-Jars-2 xDC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | HOURLY OPERATIONS SUMMARY (0000 – 2400) |  |  |  |  |  |  |  |  |  | # SURVEYS |  |  |  |  |  |  |  |  |  | # CHEMICALS USED |  |  |  |  |  |  |
|  |  | From | To |  |  | Operation |  |  |  |  |  | 1½º N60E |  | ° at |  | 1354 |  |  | m |  |  | Type |  |  | Amount |  |  |  |
|  |  | 0000 | 02.00 |  |  | Continued trip in hole to 2500m no fill or resistance |  |  |  |  |  | 4ºN340W |  | ° at |  | 1768 |  |  | m |  |  | AMC Pac L |  |  |  |  |  |  |
|  |  | 02.00 | 06.30 |  |  | Circulated and conditioned mud properties ( Mud Wt ) |  |  |  |  |  | 12ºN340W |  | ° at |  | 2093 |  |  | m |  |  | AMC Pac R |  |  |  |  |  |  |
|  |  | 06.30 | 13.00 |  |  | Tripped out for logging re-run |  |  |  |  |  | 18.5 |  | ° at |  | 2235 |  |  | m |  |  | Lakes Pac R |  |  |  |  |  |  |
|  |  | 13.00 | 16.00 |  |  | Rigged up Schlumberger-safety meeting-Ran sonic tool |  |  |  |  |  | N340W |  | ° at |  |  |  |  | m |  |  | Xan Bore |  |  | 7 sx |  |  |  |
|  |  | 16.00 | 17.00 |  |  | Worked stuck tools free at 2,000m |  |  |  |  |  |  |  | ° at |  |  |  |  | m |  |  | Bayrtes |  |  | 513 |  |  |  |
|  |  | 17.00 | 20.00 |  |  | Continued logging after freeing tools-pulled tools |  |  |  |  |  |  |  | ° at |  |  |  |  | m |  |  | Potassium Chloride |  |  |  |  |  |  |
|  |  | 20.00 | 24.00 |  |  | Ran MSCT log |  |  |  |  |  |  |  | ° at |  |  |  |  | m |  |  | AMC Biocide G |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | # PUMP 1 Duplex |  |  |  |  |  |  |  |  |  | # PUMP 2 Triplex |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Liner |  |  |  | 140 |  |  | mm |  |  | Liner |  |  | 140 |  | mm |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Pressure |  |  |  |  |  |  | kPa |  |  | Pressure |  |  | 4,300 |  | KPa |  |
|  |  |  |  |  |  |  |  |  |  |  |  | A.V. DP |  |  |  |  |  |  | f/min |  |  | A.V. DP |  |  | 215 |  | Ft/min |  |
|  |  |  |  |  |  |  |  |  |  |  |  | A.V. DC |  |  |  |  |  |  | f/min |  |  | A.V. DC |  |  | 364 |  | Ft//mn |  |
|  |  |  |  |  |  |  |  |  |  |  |  | SPM |  |  |  |  |  |  |  |  |  | SPM |  |  | 100 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | GPM |  |  |  |  |  |  |  |  |  | GPM |  |  | 225 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | # CEMENT DATA |  |  |  |  |  |  |  |  |  | # LAST CASING |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Sacks Halliburton |  |  |  |  |  |  |  |  |  | Size |  | 7" |  |  | inch |  |
|  |  |  |  |  |  | Hunt 16 camp 4 |  |  |  |  |  | Class Bulk |  |  |  |  |  |  |  |  |  | Weight |  | 26 |  |  | ppf |  |
|  |  |  |  |  |  | Lakes 4 |  |  |  |  |  | % |  |  |  |  |  |  |  |  |  | Grade |  | K-55 |  |  |  |  |
|  |  |  |  |  |  | Mud logger 2 |  |  |  |  |  | % |  |  |  |  |  |  |  |  |  | Connection |  | BTC |  |  |  |  |
|  |  |  |  |  |  | Farley Riggs 4 |  |  |  |  |  | % |  |  |  |  |  |  |  |  |  | Joints |  | 117 |  |  |  |  |
|  |  |  |  |  |  | RMN 1 |  |  |  |  |  | % |  |  |  |  |  |  |  |  |  | Length |  | RT-Brd Hd 5.0 |  |  | m |  |
|  |  | Days since | LTA |  |  | Schlumberger 6 |  |  |  |  |  | Av. Wt |  |  |  |  |  |  |  |  |  | Shoe |  | 1376m from RKB |  |  | m |  |

,66