MEGASCOLIDES No.1 Re-Entry ST1 PEP 162 / EL 4537 DAILY GEOLOGICAL REPORT No. 13 DATE: 26-12-2006

Depth: 1980m Progress: 72m Days from Spud: 13

Rig: Century Rig No.11 GL(AHD): 120.0m Drilling Rep: Bruce Pilat RT: (datum) 125.2m Geologist: David Horner Last Casing: 244mm at 505m

0600 hrs Update: Rig up Precision Logging. Run#1 DLL-SLL-MLL-GR-CSS-PDS-CNS-SP-Cal.

Comments: Drill ahead with 216mm hole to 1980m. Reached T.D. (1980m) at 1045hrs, 26th December, 2006. Circulate sample, wiper trip, POOH to log, rig up Precision Logging for Run#1 DLL-SLL-MLL-GR-CSS-PDS-CNS-SP-Cal.

| Interval (mRT) | Hydrocarbon Show Summary | Gas |
| --- | --- | --- |
| 1908-1944 | Crayfish Group - OIL SHOW From 1915-1925 the calcite vein infill material has trace to 5% dull patchy pale yellow fluorescence giving a very weak dull yellowish white crush cut, trace residue. The cutting samples would indicate these fractures to have insufficient open volume to have significant oil recovery potential. No mud losses were recorded whilst drilling this interval. | TG 0.01-0.056% C1 51% C2 18% C3 18% C4 11% C5 2% |
| 1944-1959 | Weathered Duck Bay Volcanics - No Show | TG 0.010-0.013% C1 64% C2 6% C3 9% C4 21% C5 0 |
| 1959-1980 (T.D.) | Duck Bay Volcanics - Oil Show The vein infill material (trace to 1% of sample) from 1959-1980m has trace to 40% dull to bright solid to patchy yellow fluorescence giving a weak yellow white crush cut, thin yellow ring residue. The cutting samples indicate these fractures have insufficient open volume for significant oil recovery. No mud losses were recorded whilst drilling this interval. | TG 0.01%-0.013% C1 64% C2 6% C3 9% C4 21% C5 0 |

| Formation Tops: | Prognosed (mRT) | Actual* (mRT) | Actual* (mSS) | Difference* (High/Low) |
| --- | --- | --- | --- | --- |
| Thorpdale Volcanics Barracouta Formation Childers Formation Wonthaggi Formation (Strzelecki Group) Intra Strzelecki Sands "A" Unit Intra Strzelecki Sands "B" Unit Intra Strzelecki Sands "C" Unit Intra Strzelecki Sands "D" Unit Rintoul Creek | 5.2 39 62 859 1079 1373 1726 1884 | 5.2 39 62 859 1079 1373 1726 1883.4 | +120 +86.3 +63.3 -733.7 -953.7 -1247.7 -1600.7 -1758.2 | - - - - - - 0 0.6 High |
| Crayfish Group | 1891 | 1890 | -1765 | 1 High |
| Weathered Duck Bay Volcanics | 1943 | 1944 | -1819 | 1 Low |
| Duck Bay Volcanics | 1963 | 1959 | -1834 | 4 High |
| Total Depth | 1980 | 1980 | -1855 | 0 |

*Provisional, based on mudlog

MEGASCOLIDES No.1 Re-Entry ST1 PEP 162 / EL 4537 DAILY GEOLOGICAL REPORT No. 13 DATE: 26-12-2006

| Lithological and Fluorescence Description |  |
| --- | --- |
| Interval (m) | Description |
| 1908-1944 | CRAYFISH GROUP Shale (100%) with minor laminated Sandstone (trace). SHALE: dark to very dark grey to dark brown grey, slightly to occasionally very silty, occasionally very finely arenaceous, moderately carbonaceous, trace to common fine black carbonaceous matter, trace to common white crystalline vein infill, common micromica, hard, subfissile. SANDSTONE: light grey to light brown grey, very fine to very coarse, dominantly very fine, subangular to subrounded, very poorly sorted, strong silica cement, moderate calcareous cement, trace white argillaceous matrix, quartzose, trace dark grey lithics, trace black coaly material, trace crystalline vein infill, hard, no visual porosity, no oil fluorescence. FLUORESCENCE: (1915-1925m) The vein infill material has trace to 5% dull patchy pale yellow fluorescence giving a very weak dull yellowish white crush cut, trace residue. |
| 1944-1959 | WEATHERED DUCK BAY VOLCANICS Weathered Volcanics (Basalt?) (100%). VOLCANICS: mainly weathered to off white to light brown to dark grey claystone, cryptocrystalline texture, remnant flow banding, hard, brittle, where less weathered is bright green to black, cryptocrystalline, trace calcite veining (no oil fluorescence), hard, brittle. |
| 1959-1980 T.D. | DUCK BAY VOLCANICS (100%) Massive Volcanics (100%), Basalt(?), partially weathered, in general becoming less weathered with depth. VOLCANICS: medium to dark green to black speckled with very fine spots of white clay, light brown grey and weathered in part, vesicular in part, micro to cryptocrystalline, chloritic?, common white and clear crystalline veining, hard. FLUORESCENCE: The calcite vein infill material (1% of sample) has trace to 40% dull to bright solid to patchy yellow fluorescence giving a weak yellow white crush cut, thin yellow ring residue. |