LAKES OIL N.L

DAILY DRILLING REPORT

| WELL : | Hazelwood-1 | REPORT No. : | 15 | DFS: | 14 | DATE : | 7-04-07 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2400 DEPTH : | 1262m | LAST DEPTH : | 1298m | 24 HR PROGRESS: | 36m | PTD : | 2,700 |
| TOTAL FUEL : | 18,600 | FUEL USAGE : | 3,500 | DAYCOST : $ |  | CUM. COST : $ |  |
| 0600 OPS : | Circ-ran survey –pumped pill-tripped out for casing –laying out 8in drill collars at 06:00am |  |  |  |  |  |  |
| Next 24 Hrs Op | Rig up for casing –run casing - cement same |  |  |  |  |  |  |
| Site Personnel: | Lou DeVattimo - Supervisor |  |  |  |  |  |  |

|  |  | BIT INFORMATION |  |  |  |  |  |  |  | # OPS BREAKDOWN |  |  |  |  |  |  |  |  |  |  | # MUD PROPERTIES |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Bit Number |  |  | 2 |  | 3 |  |  | Drilling |  |  |  |  | 14.00 |  |  |  |  |  | Density |  |  | 1.128 |  |  | S.G. |  |  |
|  |  | Size (mm) |  |  | 311mm |  | 311mm |  |  | Tripping |  |  |  |  | 5.50 |  |  |  |  |  | Viscosity |  |  | 49 |  |  | Sec |  |  |
|  |  | Make |  |  | Reed |  | Reed |  |  | Surveys |  |  |  |  | 1.50 |  |  |  |  |  | Water Loss |  |  | 6.6 |  |  | Cc |  |  |
|  |  | Type |  |  | DX8-140A |  | M4300-PDH |  |  | Sip & Cut block line |  |  |  |  | 0.50 |  |  |  |  |  | pH |  |  | 9.0 |  |  | Strip |  |  |
|  |  | IADC Code |  |  | S323 |  | 4-3-7 |  |  | Wash and ream |  |  |  |  |  |  |  |  |  |  | Filter Cake |  |  | 1 |  |  | 32 nd |  |  |
|  |  | Serial Number |  |  | 206579 |  | BN4133 |  |  | Circulate Sample |  |  |  |  |  |  |  |  |  |  | PV / YP |  |  | 14/ 22 |  |  | Cp/lb/100’ |  |  |
|  |  | Nozzles (32) |  |  | 7 x 14 |  | 20-20-20 |  |  | Casing |  |  |  |  |  |  |  |  |  |  | Gels |  |  | 4 / 6 |  |  | S/m |  |  |
|  |  | Depth in (m) |  |  | 556 |  | 1070 |  |  | Cementing |  |  |  |  |  |  |  |  |  |  | Sand |  |  | tr |  |  | % Vol |  |  |
|  |  | Depth out (m) |  |  | 1070 |  | 1298 |  |  | Surveys |  |  |  |  |  |  |  |  |  |  | Solids |  |  | 7.5 |  |  | % Vol |  |  |
|  |  | Total Metres |  |  | 514 |  | 228 |  |  | Circulate & Condition |  |  |  |  | 2.00 |  |  |  |  |  | Oil |  |  |  |  |  | % Vol |  |  |
|  |  | Hours |  |  | 90.5 |  |  |  |  | Rig Service |  |  |  |  | 0.50 |  |  |  |  |  | K+ |  |  |  |  |  | Mg/l |  |  |
|  |  | WOB (kg) |  |  | 12,000 |  | 12-15000 |  |  | Pick up BHA |  |  |  |  |  |  |  |  |  |  | Chlorides |  |  | 1,050 |  |  | Mg/l |  |  |
|  |  | RPM |  |  | 80 |  | 80 |  |  | Drill out Cmt |  |  |  |  |  |  |  |  |  |  | PHPA |  |  | 0.20 |  |  |  |  |  |
|  |  | Condition |  |  | Not reusable |  |  |  |  | Repair rig |  |  |  |  |  |  |  |  |  |  | KCL |  |  |  |  |  |  |  |  |
|  |  | Impact Force |  |  | 656 |  | 613 |  |  | Wait On Cement |  |  |  |  |  |  |  |  |  |  | Daily Cost $ |  |  | 3,595 |  |  |  |  |  |
|  |  | Jet Velocity |  |  | 263 |  | 234 |  |  | Test BOPs |  |  |  |  |  |  |  |  |  |  | Cum Cost $ |  |  | 33,114 |  |  |  |  |  |
|  |  | B.H.A.: 3 |  | 311mm Bit-Bit Sub-N.B.S-2 x 8” Drill Collars- Stabilizer-Crossover sub- NMDC- 8 x 6-1/4”DC- Drilling Jars- 2 x 6-1/4DC-4 HWDP |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | HOURLY OPERATIONS SUMMARY (0000 – 2400) |  |  |  |  |  |  |  |  |  | # SURVEYS |  |  |  |  |  |  |  |  |  |  | # CHEMICALS USED |  |  |  |  |  |  |
|  |  | From | To |  |  | Operation |  |  |  |  |  | 4-1/4 |  | ° at |  | 1040 |  |  |  | m |  |  | Type |  |  | Amount |  |  |  |
|  |  | 0000 | 11:00 |  |  | Drilled 311mm hole from 1262m to 1288m |  |  |  |  |  | 15 |  | ° at |  | 1276 |  |  |  | m |  |  | Aus-Gel |  |  |  |  |  |  |
|  |  | 11:00 | 11:30 |  |  | Serviced The Rig |  |  |  |  |  | 11 deg |  | ° at |  | 357 |  |  |  | m |  |  | Dextrid |  |  |  |  |  |  |
|  |  | 11:30 | 12:30 |  |  | Ran survey on wireline ( mis-run ) |  |  |  |  |  | 9 deg |  | ° at |  | 414 |  |  |  | m |  |  | Pac - R |  |  | 11 |  |  |  |
|  |  | 12:30 | 15:30 |  |  | Drilled 311mm hole from 1288m to 1298m |  |  |  |  |  | 3 deg |  | ° at |  | 471 |  |  |  | m |  |  | Barite |  |  | 174 |  |  |  |
|  |  | 15:30 | 16:30 |  |  | Circulated bottoms up |  |  |  |  |  | 2 deg |  | ° at |  | 546 |  |  |  | m |  |  | Bulk Barite |  |  |  |  |  |  |
|  |  | 16:30 | 17:00 |  |  | Ran survey at 1276m 15deg S 25deg E |  |  |  |  |  | 2 deg |  | ° at |  | 581 |  |  |  | m |  |  | PHPA/ KCL |  |  |  |  |  |  |
|  |  | 17:00 | 18:00 |  |  | Pumped 2 pills first one not heavy enough |  |  |  |  |  | 1.5 deg |  | ° at |  | 742 |  |  |  | m |  |  | Caustic Soda |  |  |  |  |  |  |
|  |  | 18:00 | 20:00 |  |  | Wiper tripped to shoe hole good |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 20:00 | 20:30 |  |  | Slipped block line |  |  |  |  |  | # PUMP 1 |  |  |  |  |  |  |  |  |  |  | # PUMP 2 |  |  |  |  |  |  |
|  |  | 20;30 | 24:00 |  |  | Tripped back in hole washed down last 2 singles no fill |  |  |  |  |  | Liner |  |  |  | 108 |  | mm |  |  |  |  | Liner |  |  | 108 |  | mm |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Pressure |  |  |  | 7,700 |  | kPa |  |  |  |  | Pressure |  |  | 7,700 |  | KPa |  |
|  |  |  |  |  |  |  |  |  |  |  |  | A.V. DP |  |  |  | 103 |  | f/min |  |  |  |  | A.V. DP |  |  | 103 |  | f/min |  |
|  |  |  |  |  |  |  |  |  |  |  |  | A.V. DC |  |  |  | 155 |  | f/min |  |  |  |  | A.V. DC |  |  | 155 |  | f/min |  |
|  |  |  |  |  |  |  |  |  |  |  |  | SPM |  |  |  | 130 |  |  |  |  |  |  | SPM |  |  | 130 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | GPM |  |  |  | 272 |  |  |  |  |  |  | GPM |  |  | 272 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | # CEMENT DATA |  |  |  |  |  |  |  |  |  |  | # LAST CASING |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Sacks Halliburton |  |  |  |  |  |  |  |  |  |  | Size |  | 340 |  |  | mm |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Class Bulk |  |  |  |  |  |  |  |  |  |  | Weight |  | 81 |  |  | Kg/m |  |
|  |  |  |  |  |  |  |  |  |  |  |  | % |  |  |  |  |  |  |  |  |  |  | Grade |  | K-55 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | % |  |  |  |  |  |  |  |  |  |  | Connection |  | B.T.C |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | % |  |  |  |  |  |  |  |  |  |  | Joints |  | 38 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | % |  |  |  |  |  |  |  |  |  |  | Length |  | 420.00 |  |  | m |  |
|  |  | Days since | LTA |  |  |  |  |  |  |  |  | Av. Wt |  |  |  |  |  |  |  |  |  |  | Shoe |  | 420 m from RKB |  |  | m |  |

,66