LAKES OIL N.L

DAILY DRILLING REPORT

| WELL : | Hazelwood-1 | REPORT No. : | 20 | DFS: | 19 | DATE : | 12-04-07 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2400 DEPTH : | 1379m | LAST DEPTH : | 1335m | 24 HR PROGRESS: | 44m | PTD : | 2,700 |
| TOTAL FUEL : | 11,00 | FUEL USAGE : | 2,600 | DAYCOST : $ |  | CUM. COST : $ |  |
| 0600 OPS : | Drilled 216mm hole from 1379m to 1431m |  |  |  |  |  |  |
| Next 24 Hrs Op | Drill ahead - survey at 1440m |  |  |  |  |  |  |
| Site Personnel: | Lou DeVattimo - Supervisor |  |  |  |  |  |  |

|  |  | BIT INFORMATION |  |  |  |  |  |  |  | # OPS BREAKDOWN |  |  |  |  |  |  |  |  |  |  | # MUD PROPERTIES |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Bit Number |  |  | 4 |  |  |  |  | Drilling |  |  |  |  | 9.50 |  |  |  |  |  | Density |  |  | 1.092 |  |  | S.G. |  |  |
|  |  | Size (mm) |  |  | 216 |  |  |  |  | Tripping |  |  |  |  | 11.50 |  |  |  |  |  | Viscosity |  |  | 35 |  |  | Sec |  |  |
|  |  | Make |  |  | Reed |  |  |  |  | Surveys |  |  |  |  | 0.50 |  |  |  |  |  | Water Loss |  |  | 8.00 |  |  | Cc |  |  |
|  |  | Type |  |  | DSX-104GH |  |  |  |  | Sip & Cut block line |  |  |  |  | 1.50 |  |  |  |  |  | pH |  |  | 11.0 |  |  | Strip |  |  |
|  |  | IADC Code |  |  | M 223 |  |  |  |  | Wash and ream |  |  |  |  |  |  |  |  |  |  | Filter Cake |  |  | 1 |  |  | 32 nd |  |  |
|  |  | Serial Number |  |  | 109683 |  |  |  |  | Circulate Sample |  |  |  |  |  |  |  |  |  |  | PV / YP |  |  | 9 / 8 |  |  | Cp/lb/100’ |  |  |
|  |  | Nozzles (32) |  |  | 5 x 14 |  |  |  |  | Casing |  |  |  |  |  |  |  |  |  |  | Gels |  |  | 2 / 3 |  |  | S/m |  |  |
|  |  | Depth in (m) |  |  | 1298 |  |  |  |  | Cementing |  |  |  |  |  |  |  |  |  |  | Sand |  |  | tr |  |  | % Vol |  |  |
|  |  | Depth out (m) |  |  |  |  |  |  |  | Surveys |  |  |  |  |  |  |  |  |  |  | Solids |  |  | 5.3 |  |  | % Vol |  |  |
|  |  | Total Metres |  |  | 81 |  |  |  |  | Circulate & Condition |  |  |  |  | 1.00 |  |  |  |  |  | Oil |  |  |  |  |  | % Vol |  |  |
|  |  | Hours |  |  | 14.50 |  |  |  |  | Rig Service |  |  |  |  |  |  |  |  |  |  | K+ |  |  |  |  |  | Mg/l |  |  |
|  |  | WOB (kg) |  |  | 3,000 |  |  |  |  | Make up BHA |  |  |  |  |  |  |  |  |  |  | Chlorides |  |  | 1,800 |  |  | Mg/l |  |  |
|  |  | RPM |  |  | 100 |  |  |  |  | Drill out Cmt |  |  |  |  |  |  |  |  |  |  | PHPA |  |  | 0.15 |  |  |  |  |  |
|  |  | Condition |  |  |  |  |  |  |  | Repair rig |  |  |  |  |  |  |  |  |  |  | KCL |  |  |  |  |  |  |  |  |
|  |  | Impact Force |  |  | 352 |  |  |  |  | Wait On Cement |  |  |  |  |  |  |  |  |  |  | Daily Cost $ |  |  | 3,072 |  |  |  |  |  |
|  |  | Jet Velocity |  |  | 178 |  |  |  |  | F.I.T |  |  |  |  |  |  |  |  |  |  | Cum Cost $ |  |  | 42,556 |  |  |  |  |  |
|  |  | B.H.A.: 4 |  | 216mm PDC Bit-.Bit Sub-NMDC- Stab-1 x 6-1/4” Drill Collar-Stabilizer-12 x 6-1/4” Drill Collars-Jars- 2 x 6-1/4’ Drill Collars- 4 x HWDP |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | HOURLY OPERATIONS SUMMARY (0000 – 2400) |  |  |  |  |  |  |  |  |  | # SURVEYS |  |  |  |  |  |  |  |  |  |  | # CHEMICALS USED |  |  |  |  |  |  |
|  |  | From | To |  |  | Operation |  |  |  |  |  | 5 |  | ° at |  | 841 |  |  |  | m |  |  | Type |  |  | Amount |  |  |  |
|  |  | 0000 | 03;30 |  |  | Drilled 216mm hole from 1335m to 1350m |  |  |  |  |  | 3 |  | ° at |  | 944 |  |  |  | m |  |  | Aus-Gel |  |  |  |  |  |  |
|  |  | 03;30 | 04;00 |  |  | Ran survey on wireline |  |  |  |  |  | 6.5 |  | ° at |  | 1140 |  |  |  | m |  |  | Pac L |  |  |  |  |  |  |
|  |  | 04:00 | 04:30 |  |  | Drilled 216mm hole from 1350m to 1353m |  |  |  |  |  | 15 |  | ° at |  | 1268 |  |  |  | m |  |  | Pac - R |  |  | 17 |  |  |  |
|  |  | 04:30 | 05:30 |  |  | Circulated hole clean prior to trip out for BHA change |  |  |  |  |  | 20 |  | ° at |  | 1340 |  |  |  | m |  |  | Barite |  |  | 40 |  |  |  |
|  |  | 05;30 | 10:00 |  |  | Tripped out of hole |  |  |  |  |  |  |  | ° at |  |  |  |  |  | m |  |  | Citric acid |  |  |  |  |  |  |
|  |  | 10:00 | 10:30 |  |  | Changed position of stabilizer picked up one drill collar |  |  |  |  |  |  |  | ° at |  |  |  |  |  | m |  |  | PHPA/ KCL |  |  |  |  |  |  |
|  |  | 10:30 | 11:30 |  |  | Tripped back in hole to 450m |  |  |  |  |  |  |  | ° at |  |  |  |  |  | m |  |  | Caustic Soda |  |  |  |  |  |  |
|  |  | 11:30 | 12:30 |  |  | Pulled back out to re-arrange BHA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 12:30 | 13:00 |  |  | Changed BHA stabilizer positions |  |  |  |  |  | # PUMP 1 |  |  |  |  |  |  |  |  |  |  | # PUMP 2 |  |  |  |  |  |  |
|  |  | 13;00 | 16:30 |  |  | Tripped in hole to 1291m |  |  |  |  |  | Liner |  |  |  | 108 |  | mm |  |  |  |  | Liner |  |  | 108 |  | mm |  |
|  |  | 16:30 | 18:00 |  |  | Slipped and cut block line |  |  |  |  |  | Pressure |  |  |  | 5,600 |  | kPa |  |  |  |  | Pressure |  |  | 5,600 |  | KPa |  |
|  |  | 18;00 | 18;30 |  |  | Tripped in hole to1353m |  |  |  |  |  | A.V. DP |  |  |  | 197 |  | f/min |  |  |  |  | A.V. DP |  |  | 197 |  | f/min |  |
|  |  | 18;30 | 24:00 |  |  | Drilled 216mm hole from 1353m to `1379m |  |  |  |  |  | A.V. DC |  |  |  | 309 |  | f/min |  |  |  |  | A.V. DC |  |  | 309 |  | f/min |  |
|  |  |  |  |  |  |  |  |  |  |  |  | SPM |  |  |  | 100 |  |  |  |  |  |  | SPM |  |  | 100 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | GPM |  |  |  | 210 |  |  |  |  |  |  | GPM |  |  | 210 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | # CEMENT DATA |  |  |  |  |  |  |  |  |  |  | # LAST CASING |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Sacks Halliburton |  |  |  |  | 900 |  |  |  |  |  | Size |  | 244 |  |  | mm |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Class Bulk |  |  |  |  |  |  |  |  |  |  | Weight |  | 36 |  |  | Lb/ft |  |
|  |  |  |  |  |  |  |  |  |  |  |  | % |  |  |  |  |  |  |  |  |  |  | Grade |  | K-55 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | % |  |  |  |  |  |  |  |  |  |  | Connection |  | B.T.C |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | % |  |  |  |  |  |  |  |  |  |  | Joints |  | 110 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | % |  |  |  |  |  |  |  |  |  |  | Length |  | 1291.00 |  |  | m |  |
|  |  | Days since | LTA |  |  |  |  |  |  |  |  | Av. Wt |  |  |  |  |  |  |  |  |  |  | Shoe |  | 1296m from RKB |  |  | m |  |

,66