PETRO TECH PTY. LTD. A.C.N. (009 116 429) HAZELWOOD No.1 PEP 166

DAILY GEOLOGICAL REPORT No. 1 Date: 24-03-2007

Depth: 293m Progress: 0m Days from Spud: 1

Rig: CENTURY RIG No.11 GL(AHD): 100.0m Drilling Rep: Lou DeVattimo RT: (datum) 105.2m Geologist: David Horner Last Casing: 18" at 286m

0600 hrs Update: Drill out cement in casing.

Comments: Drill and instal rathole and mousehole, make up and RIH with 16" drilling assembly, drill out cement prior to drilling ahead with 16" hole.

| Interval (mRT) | Hydrocarbon Show Summary | Gas |
| --- | --- | --- |
|  | No new formation drilled. |  |

| Formation Tops: | Prognosed (mRT) | Actual* (mRT) | Actual* (mSS) | Difference* (High/Low) |
| --- | --- | --- | --- | --- |
| Haunted Hill Formation Yallourn Formation Morwell Formation Traralgon Formation | 5.2 10 170 440 | 5.2 - - | +100 - - | 0 - - |
| Golden Beach (Kipper Shale) | 530 |  |  |  |
| Strzelecki Formation | 1030 |  |  |  |
| Rintouls Creek Sand (Tyers Conglomerate) | 2430 |  |  |  |
| Basement | 2730 |  |  |  |
| Total Depth | 2750 |  |  |  |

*Provisional, based on mudlog

| Lithological and Fluorescence Description |  |
| --- | --- |
| Interval (m) | Description |
|  | No new formation drilled. |