PETRO TECH PTY. LTD. A.C.N. (009 116 429) HAZELWOOD No.1 PEP 166

DAILY GEOLOGICAL REPORT No. 10 Date: 02-04-2007

Depth: 951m Progress: 56m Days from Spud: 10

Rig: CENTURY RIG No.11 GL(AHD): 100.0m Drilling Rep: Lou DeVattimo RT: (datum) 105.2m Geologist: David Horner Last Casing: 13⅜" at 420m

0600 hrs Update: Drill ahead with 12¼" hole to 988m. Current lithology Claystone (90%) with minor laminated tight Sandstone (10%), trace tight calcite lined fractures, background gas 0.1 unit, C1 100%.

Comments: Drill ahead with 12¼" hole to 904m, rotary chain housing broke, pull back to shoe, repair rotary drive, RIH, wiper trip gas 0.26 units, drill ahead with 12¼" hole to 951m.

| Interval (mRT) | Hydrocarbon Show Summary | Gas |
| --- | --- | --- |
| 895-951 | Strzelecki Formation - No Show | TG 0.1-0.3u C1 100% C2+ 0 |

| Formation Tops: | Prognosed (mRT) | Actual* (mRT) | Actual* (mSS) | Difference* (High/Low) |
| --- | --- | --- | --- | --- |
| Haunted Hill Formation | 5.2 | 5.2 | +100 | 0 |
| LaTrobe Group | 10 | - | - | - |
| Strzelecki Formation | 530 | 545 | -440 | 15 Low |
| Rintouls Creek Sand (Tyers Conglomerate) | 2430 |  |  |  |
| Basement | 2730 |  |  |  |
| Total Depth | 2750 |  |  |  |

*Provisional, based on mudlog

| Lithological and Fluorescence Description |  |
| --- | --- |
| Interval (m) | Description |
| 895-951 | STRZELECKI FORMATION Claystone (90%) laminated and thinly interbedded with silty Sandstone (10%). CLAYSTONE: medium dark grey to occasionally medium green grey, slightly to very silty, common dispersed very fine altered feldspar grains in part, trace black carbonaceous flecks, trace to common micromica, trace to common calcite lined fractures, hard, subfissile. SANDSTONE: medium grey to medium green grey, silty to very fine, subangular to subrounded, moderately sorted, strong silica cement, abundant white and grey argillaceous and silt matrix, abundant off white altered feldspar grains, common green grey and trace red lithics, trace fine black carbonaceous detritus, trace to common calcite lined fractures, very hard, no visual porosity, no oil fluorescence. |