PETRO TECH PTY. LTD. CUTTINGS DESCRIPTIONS WELL NAME: Hazelwood No.1 DATE: 02-04-2007 GEOLOGIST: David Horner PAGE: 1

| Interval (m) | % | Description |
| --- | --- | --- |
| 904-912 | 100 | CLAYSTONE: medium dark grey to occasionally medium green grey, slightly to very silty, common dispersed very fine altered feldspar grains in part, trace black carbonaceous flecks, trace to common micromica, trace calcite lined fractures, hard, subfissile. |
|  | Trace | SANDSTONE: medium grey to medium green grey, silty to very fine, subangular to subrounded, moderately sorted, strong silica cement, abundant white and grey argillaceous and silt matrix, abundant off white altered feldspar grains, common green grey and trace red lithics, trace fine black carbonaceous detritus, trace calcite lined fractures, very hard, no visual porosity, no oil fluorescence. |
| 912-915 | 90 | CLAYSTONE: as for 904-912m. |
|  | 10 | SANDSTONE: as for 904-912m. |
| 915-921 | 100 | CLAYSTONE: medium dark grey to occasionally medium green grey to medium brown grey, slightly to very silty, common dispersed very fine altered feldspar grains in part, trace black carbonaceous flecks, trace to common micromica, trace calcite lined fractures, hard, subfissile. |
|  | Trace | SANDSTONE: as for 904-912m. |
| 921-930 | 90 | CLAYSTONE: as for 915-921m. |
|  | 10 | SANDSTONE: as for 904-912m. |
| 930-939 | 100 | CLAYSTONE: medium dark grey to occasionally medium green grey, slightly to very silty, common dispersed very fine altered feldspar grains in part, trace black carbonaceous flecks, trace to common micromica, trace calcite lined fractures, hard, subfissile. |
|  | Trace | SANDSTONE: as for 904-912m. |
| 939-948 | 90 | CLAYSTONE: as for 930-939m. |
|  | 10 | SANDSTONE: as for 904-912m. |
| 948-960 | 100 | CLAYSTONE: medium to dark grey to occasionally medium green grey, slightly to very silty, common dispersed very fine altered feldspar grains in part, trace black carbonaceous flecks, trace to common micromica, common calcite lined fractures, hard, subfissile. |
|  | Trace | SANDSTONE: medium grey to medium green grey, silty to very fine, subangular to subrounded, moderately sorted, strong silica cement, abundant white and grey argillaceous and silt matrix, abundant off white altered feldspar grains, common green grey and trace red lithics, trace fine black carbonaceous detritus, common calcite lined fractures, very hard, no visual porosity, no oil fluorescence. |
| 960-972 | 100 | CLAYSTONE: medium to dark grey to medium green grey, slightly to very silty, common dispersed very fine altered feldspar grains in part, trace black carbonaceous flecks, trace to common micromica, common calcite lined fractures, hard, subfissile. |
|  | Trace | SANDSTONE: as for 948-960m. |
| 972-975 | 90 | CLAYSTONE: as for 960-972m. |
|  | 10 | SANDSTONE: as for 948-960m. |
| 975-984 | 100 | CLAYSTONE: medium to dark grey to medium green grey, slightly to very silty, common dispersed very fine altered feldspar grains in part, trace black carbonaceous flecks, trace to common micromica, trace calcite lined fractures, hard, subfissile. |
|  | Trace | SANDSTONE: medium grey to medium green grey, silty to very fine, subangular to subrounded, moderately sorted, strong silica cement, abundant white and grey argillaceous and silt matrix, abundant off white altered feldspar grains, common green grey and trace red lithics, trace fine black carbonaceous detritus, trace calcite lined fractures, very hard, no visual porosity, no oil fluorescence. |
| 984-987 | 90 | CLAYSTONE: as for 975-984m. |
|  | 10 | SANDSTONE: as for 975-984m. |