PETRO TECH PTY. LTD. A.C.N. (009 116 429) HAZELWOOD No.1 PEP 166

DAILY GEOLOGICAL REPORT No. 12 Date: 04-04-2007

Depth: 1118m Progress: 46m Days from Spud: 12

Rig: CENTURY RIG No.11 GL(AHD): 100.0m Drilling Rep: Lou DeVattimo RT: (datum) 105.2m Geologist: David Horner Last Casing: 13⅜" at 420m

0600 hrs Update: Drill ahead with 12¼" hole at 1139m. Current lithology - Strzelecki Formation claystone (100%) with trace very fine tight sandstone, background gas 0.2 units (C1 100%).

Comments: POOH, make up new 12¼" 437 tricone bit, RIH, drill ahead with 12¼" hole to 1118m, progressively increase mud weight to 9.2 ppg to control well bore stresses.

| Interval (mRT) | Hydrocarbon Show Summary | Gas |
| --- | --- | --- |
| 1072-1118 | Strzelecki Formation - No Show Probable fault zone present at 1068m, with hot basaltic water alteration of Strzelecki Formation below 1068m (progressively decreases with depth). Hole instability suggests a tectonically stressed zone is present, associated with and juxtaposed to this fault. | TG 0.1-0.3u C1 100% C2+ 0 |

| Formation Tops: | Prognosed (mRT) | Actual* (mRT) | Actual* (mSS) | Difference* (High/Low) |
| --- | --- | --- | --- | --- |
| Haunted Hill Formation | 5.2 | 5.2 | +100 | 0 |
| LaTrobe Group | 10 | - | - | - |
| Strzelecki Formation | 530 | 545 | -440 | 15 Low |
| Rintouls Creek Sand (Tyers Conglomerate) | 2430 |  |  |  |
| Basement | 2730 |  |  |  |
| Total Depth | 2750 |  |  |  |

*Provisional, based on mudlog

| Lithological and Fluorescence Description |  |
| --- | --- |
| Interval (m) | Description |
| 1072-1118 | STRZELECKI FORMATION Claystone (90%) with minor laminated and thinly interbedded Sandstone (10%). CLAYSTONE: medium green grey, occasionally medium grey to medium brown grey, moderately silty in part, trace to common micromica, trace to common calcite lined fractures, hard, subfissile. SANDSTONE: medium green grey, silty to very fine, subrounded, moderately to well sorted, strong silica cement, abundant light green grey siliceous matrix, abundant altered feldspar grains, common green grey and trace red lithics, trace black carbonaceous flecks, common calcite lined fractures, hard to very hard, no visual porosity, no oil fluorescence. |