PETRO TECH PTY. LTD. A.C.N. (009 116 429) HAZELWOOD No.1 PEP 166

DAILY GEOLOGICAL REPORT No. 14 Date: 06-04-2007

Depth: 1262m Progress: 64m Days from Spud: 14

Rig: CENTURY RIG No.11 GL(AHD): 100.0m Drilling Rep: Lou DeVattimo RT: (datum) 105.2m Geologist: David Horner Last Casing: 13⅜" at 420m

0600 hrs Update: Drill ahead with 12¼" hole at 1276m. Current lithology, Strzelecki Formation claystone (100%), background gas 0.1% (C1 100%).

Comments: Drill ahead with 12¼" hole to 1262m.

| Interval (mRT) | Hydrocarbon Show Summary | Gas |
| --- | --- | --- |
| 1198-1262 | Strzelecki Formation - No Show | TG 0.1-0.3u C1 100% C2+ 0 |

| Formation Tops: | Prognosed (mRT) | Actual* (mRT) | Actual* (mSS) | Difference* (High/Low) |
| --- | --- | --- | --- | --- |
| Haunted Hill Formation | 5.2 | 5.2 | +100 | 0 |
| LaTrobe Group | 10 | - | - | - |
| Strzelecki Formation | 530 | 545 | -440 | 15 Low |
| Rintouls Creek Sand (Tyers Conglomerate) | 2430 |  |  |  |
| Basement | 2730 |  |  |  |
| Total Depth | 2750 |  |  |  |

*Provisional, based on mudlog

| Lithological and Fluorescence Description |  |
| --- | --- |
| Interval (m) | Description |
| 1198-1262 | STRZELECKI FORMATION Claystone (100%) with minor laminated and thinly interbedded Sandstone (trace). CLAYSTONE: medium to dark green grey to medium grey to dark brown grey, moderately silty in part, trace very fine altered feldspars in part, trace very fine black cabonaceous flecks in part, trace to common micromica, trace calcite lined fractures, hard, subfissile. SANDSTONE: light brown to medium green grey, silty to very fine, subrounded, moderately to well sorted, strong silica cement, abundant light green grey siliceous matrix, abundant altered feldspar grains, common green grey and trace red lithics, trace black carbonaceous flecks, trace calcite lined fractures, hard to very hard, no visual porosity, no oil fluorescence. |