PETRO TECH PTY. LTD. A.C.N. (009 116 429) HAZELWOOD No.1 PEP 166

DAILY GEOLOGICAL REPORT No. 19 Date: 11-04-2007

Depth: 1335m Progress: 37m Days from Spud: 19

Rig: CENTURY RIG No.11 GL(AHD): 100.0m Drilling Rep: Lou DeVattimo RT: (datum) 105.2m Geologist: David Horner Last Casing: 9⅝" at 1298m

0600 hrs Update: Drill ahead with 8½" hole to 1353m, survey 20 degrees at 88Az, POOH for BHA change. Current lithology 90% claystone laminated with 10% very fine tight sandstone, background gas 0.2 units (C1 100%).

Comments: Drill out cement from 853-1298m, drill ahead with 8½" hole to 1302m, perform PIT to 1100 PSI - no leak off (EMW 14.1 ppg), drill ahead with 8½" hole to 1335m.

| Interval (mRT) | Hydrocarbon Show Summary | Gas |
| --- | --- | --- |
| 1298-1335 | Strzelecki Formation - No Show. | TG 0.1-0.3u C1 100% C2+ 0 |

| Formation Tops: | Prognosed (mRT) | Actual* (mRT) | Actual* (mSS) | Difference* (High/Low) |
| --- | --- | --- | --- | --- |
| Haunted Hill Formation | 5.2 | 5.2 | +100 | 0 |
| LaTrobe Group | 10 | - | - | - |
| Strzelecki Formation | 530 | 545 | -440 | 15 Low |
| Rintouls Creek Sand (Tyers Conglomerate) | 2430 |  |  |  |
| Basement | 2730 |  |  |  |
| Total Depth | 2750 |  |  |  |

*Provisional, based on mudlog

| Lithological and Fluorescence Description |  |
| --- | --- |
| Interval (m) | Description |
| 1298-1335 | STRZELECKI FORMATION Claystone (80%) with minor laminated Sandstone (20%). CLAYSTONE: medium grey to medium green grey, non to occasionally very silty, occasionally very finely arenaceous with altered feldspar grains, trace very fine black carbonaceous flecks in part, common micromica, common calcite lined fractures, hard, subfissile to fissile. SANDSTONE: medium green grey, silty to very fine, subrounded, moderately to well sorted, very strong silica and weak calcareous cements, abundant green siliceous matrix, common to abundant altered feldspar grains, trace black carbonaceous flecks, common calcite lined fractures, very hard, no visual porosity, no oil fluorescence. |