PETRO TECH PTY. LTD. CUTTINGS DESCRIPTIONS WELL NAME: Hazelwood No.1 DATE: 25-03-2007 GEOLOGIST: David Horner PAGE: 1

| Interval (m) | % | Description |
| --- | --- | --- |
| 280-290 | 100 | Cement |
| 290-300 | 100 | SANDSTONE: light brown, very fine to grit, dominantly very coarse to grit, angular to subangular, moderately to well sorted, very weak silica cement, trace medium to dark brown argillaceous matrix, clear to opaque quartz grains, trace black coal detritus, rare pyrite, unconsolidated, very good inferred porosity, no oil fluorescence. |
| 300-310 | 80 | SANDSTONE: light brown, very fine to grit, dominantly very coarse, angular to subangular, poorly sorted, very weak silica cement, trace medium to dark brown argillaceous matrix, clear to opaque quartz grains, trace black coal detritus, rare pyrite, unconsolidated, very good inferred porosity, no oil fluorescence. |
|  | 20 | COAL: very dark brown to black, slightly to moderately argillaceous, irregular fracture, earthy lustre, firm to hard. |
| 310-325 | 100 | BASALT: (weathered), light to medium green grey to light brown, speckled, microcrystalline, weathered in part to kaolin, hard to very hard. |
| 325-330 | 100 | SANDSTONE: light brown, very fine to very coarse, dominantly coarse, angular to subrounded, moderately sorted, very weak silica cement, trace medium to dark brown argillaceous matrix, clear to opaque quartz grains, trace black coal detritus, unconsolidated, very good inferred porosity, no oil fluorescence. |
| 330-335 | 100 | SANDSTONE: light brown, very fine to grit, dominantly very coarse, angular to subangular, moderately sorted, very weak silica cement, trace medium to dark brown argillaceous matrix, clear to opaque quartz grains, trace black coal detritus, unconsolidated, very good inferred porosity, no oil fluorescence. |
| 335-340 | 60 | SANDSTONE: as for 330-335m. |
|  | 20 | CLAYSTONE: light to medium grey to medium brown, very carbonaceous in part, occasionally very finely arenaceous, very soft, very dispersive, non fissile. |
|  | 20 | COAL: very dark brown to black, slightly to moderately argillaceous, irregular fracture, earthy lustre, firm to hard. |
| 340-345 | 100 | SANDSTONE: very light grey to light brown grey, very fine to grit, dominantly very coarse, angular to subangular, moderately sorted, very weak silica cement, trace medium to dark brown argillaceous matrix, clear to opaque quartz grains, trace black coal detritus, trace coarse mica flakes, unconsolidated, very good inferred porosity, no oil fluorescence. |
|  | Trace | COAL: as for 335-340m. |
| 345-360 | 90 | SANDSTONE: as for 340-345m. |
|  | 10 | CLAYSTONE: as for 335-340m. |
| 360-370 | 100 | SANDSTONE: light brown grey, very fine to grit, dominantly very coarse, angular to subangular, moderately sorted, very weak silica cement, trace medium to dark brown argillaceous matrix, clear to opaque quartz grains, trace black coal detritus, unconsolidated, very good inferred porosity, no oil fluorescence. |
|  | Trace | COAL: as for 335-340m. |
| 370-375 | 80 | SANDSTONE: light brown grey, very fine to pebble, dominantly very coarse, angular to subangular, moderately sorted, very weak silica cement, trace medium to dark brown argillaceous matrix, clear to opaque quartz grains, trace black coal detritus, unconsolidated, very good inferred porosity, no oil fluorescence. |
|  | 10 | CLAYSTONE: light to medium grey to dark brown grey, very carbonaceous in part, occasionally very finely arenaceous, very soft, very dispersive, non fissile. |
|  | 10 | COAL: very dark brown to black, slightly to moderately argillaceous, irregular fracture, earthy lustre, firm to hard. |
| 375-385 | 70 | SANDSTONE: light brown grey, very fine to grit, dominantly very coarse, angular to subangular, moderately sorted, very weak silica cement, trace to common medium to dark brown argillaceous matrix, clear to opaque quartz grains, trace black coal detritus, unconsolidated, very good inferred porosity, no oil fluorescence. |
|  | 10 | CLAYSTONE: as for 370-375m. |
|  | 20 | COAL: as for 370-375m. |
| 385-400 | 100 | SANDSTONE: light brown grey, very fine to grit, dominantly very coarse, angular to subangular, moderately sorted, very weak silica cement, trace to common medium to dark brown argillaceous matrix, clear to opaque quartz grains, common black coal detritus, trace coarse mica flakes, unconsolidated, very good inferred porosity, no oil fluorescence. |
| 400-420 | 100 | SANDSTONE: light brown grey, very fine to grit, dominantly very coarse, angular to subangular, moderately sorted, very weak silica cement, trace to common medium to dark brown argillaceous matrix, clear to opaque quartz grains, trace orange red lithics, common black coal detritus, trace coarse mica flakes, unconsolidated, very good inferred porosity, no oil fluorescence. |
| 420-435 | 100 | SANDSTONE: light brown grey, very fine to pebble, dominantly very coarse, angular to subangular, moderately sorted, very weak silica cement, trace to common medium to dark brown argillaceous matrix, clear to opaque quartz grains, trace orange red lithics, common black coal detritus, trace coarse mica flakes, unconsolidated, very good inferred porosity, no oil fluorescence. |
| 435-445 | 100 | SANDSTONE: as for 400-420m. |
|  | Trace | CLAYSTONE: light to medium grey to dark brown grey, very carbonaceous in part, occasionally very finely arenaceous, very soft, very dispersive, non fissile. |
| 445-450 | 60 | SANDSTONE: light brown grey, very fine to very coarse, dominantly coarse, angular to subangular, moderately sorted, very weak silica cement, trace to common medium to dark brown argillaceous matrix, clear to opaque quartz grains, trace orange red lithics, common black coal detritus, trace coarse mica flakes, unconsolidated, very good inferred porosity, no oil fluorescence. |
|  | 40 | CLAYSTONE: light to medium grey, occasionally very finely arenaceous, rare pyrite, very soft, very dispersive, non fissile. |
| 450-460 | 90 | SANDSTONE: light brown grey, very fine to grit, dominantly very coarse, angular to subangular, moderately sorted, very weak silica cement, trace to common medium to dark brown argillaceous matrix, clear to opaque quartz grains, trace orange red lithics, common black coal detritus, trace coarse mica flakes, rare pyrite, unconsolidated, very good inferred porosity, no oil fluorescence. |
|  | 10 | CLAYSTONE: as for 445-450m. |
| 460-490 | 100 | SANDSTONE: as for 450-460m. |
| 495-500 | 90 | SANDSTONE: light brown grey, very fine to very coarse, dominantly coarse, angular to subangular, moderately sorted, very weak silica cement, trace to common medium to dark brown argillaceous matrix, clear to opaque quartz grains, trace orange red lithics, common black coal detritus, trace coarse mica flakes, rare pyrite, unconsolidated, very good inferred porosity, no oil fluorescence. |
|  | 10 | CLAYSTONE: medium grey, very finely to finely arenaceous in part, rare pyrite, very soft, very dispersive, non fissile. |
| 500-503 | 60 | SANDSTONE: light brown grey, very fine to coarse, dominantly fine to medium, angular to subangular, moderately sorted, very weak silica cement, trace to common medium to dark brown argillaceous matrix, clear to opaque quartz grains, trace orange red lithics, common black coal detritus, trace coarse mica flakes, rare pyrite, unconsolidated, very good inferred porosity, no oil fluorescence. |
|  | 40 | CLAYSTONE: as for 495-500m. |
| 503-506 | 70 | SANDSTONE: as for 500-503m. |
|  | 30 | CLAYSTONE: as for 495-500m. |
| 506-512 | 30 | SANDSTONE: as for 500-503m. |
|  | 70 | CLAYSTONE: (possible weathered or reworked volcanics?), off white to orange brown to brown, mottled texture, common dispersed very fine to very coarse yellow-red stained angular quartz grains, soft to moderately hard, moderately dispersive, non fissile. |
| 512-518 | 100 | CLAYSTONE: (possible weathered or reworked volcanics?), off white to orange brown to brown to brick red, mottled texture, trace to common dispersed very fine to very coarse yellow-red stained angular quartz grains, soft to moderately hard, moderately dispersive, non fissile. |