PETRO TECH PTY. LTD. A.C.N. (009 116 429) HAZELWOOD No.1 PEP 166

DAILY GEOLOGICAL REPORT No. 2 Date: 25-03-2007

Depth: 483m Progress: 190m Days from Spud: 2

Rig: CENTURY RIG No.11 GL(AHD): 100.0m Drilling Rep: Lou DeVattimo RT: (datum) 105.2m Geologist: David Horner Last Casing: 18" at 286m

0600 hrs Update: Drill ahead with 16" hole to 523m, repair rotary drive chain. Lithology: 483-506m LaTrobe Group sandstone. From 506-518m CLAYSTONE: (possible weathered or reworked volcanics?), off white to orange brown to brown to brick red, mottled texture, trace to common dispersed very fine to very coarse yellow-red stained angular quartz grains, soft to moderately hard, moderately dispersive, non fissile. Background gas trace C1.

Comments: Make up and RIH with 16" drilling assembly, drill out cement, drill ahead with 16" hole to 483m. Survey at 319m = 12 degrees, Survey at 357m = 10 degrees.

| Interval (mRT) | Hydrocarbon Show Summary | Gas |
| --- | --- | --- |
| 293-483 | LaTrobe Group - No Show. | TG tr-1.5u C1 100% C2+ 0 |

| Formation Tops: | Prognosed (mRT) | Actual* (mRT) | Actual* (mSS) | Difference* (High/Low) |
| --- | --- | --- | --- | --- |
| Haunted Hill Formation LaTrobe Group - Yallourn Formation LaTobe Group - Morwell Formation LaTrobe Group - Traralgon Formation | 5.2 10 170 440 | 5.2 - - - | +100 - - - | 0 - - - |
| Golden Beach (Kipper Shale) | 530 |  |  |  |
| Strzelecki Formation | 1030 |  |  |  |
| Rintouls Creek Sand (Tyers Conglomerate) | 2430 |  |  |  |
| Basement | 2730 |  |  |  |
| Total Depth | 2750 |  |  |  |

*Provisional, based on mudlog

| Lithological and Fluorescence Description |  |
| --- | --- |
| Interval (m) | Description |
| 293-318 | LATROBE GROUP Sandstone (90%) interbedded with minor Coal (10%). SANDSTONE: light brown, very fine to grit, dominantly very coarse, angular to subangular, poorly sorted, very weak silica cement, trace medium to dark brown argillaceous matrix, clear to opaque quartz grains, trace black coal detritus, rare pyrite, unconsolidated, very good inferred porosity, no oil fluorescence. COAL: very dark brown to black, slightly to moderately argillaceous, irregular fracture, earthy lustre, firm to hard. |
| 318-327 | LATROBE GROUP Weathered Basalt (100%). BASALT: (weathered), light to medium green grey to light brown, speckled, microcrystalline, weathered in part to kaolin, hard to very hard. |
| 327-385 | LATROBE GROUP Sandstone (80%) interbedded with Claystone (20%) minor Coal (trace). SANDSTONE: very light grey to light brown grey, very fine to grit, dominantly very coarse, angular to subangular, moderately sorted, very weak silica cement, trace medium to dark brown argillaceous matrix, clear to opaque quartz grains, trace black coal detritus, trace coarse mica flakes, unconsolidated, very good inferred porosity, no oil fluorescence. CLAYSTONE: light to medium grey to medium brown, very carbonaceous in part, occasionally very finely arenaceous, very soft, very dispersive, non fissile. COAL: very dark brown to black, slightly to moderately argillaceous, irregular fracture, earthy lustre, firm to hard. |
| 385-483 | LATROBE GROUP Sandstone (90%) with minor Claystone (10%). SANDSTONE: light brown grey, very fine to grit, dominantly very coarse, angular to subangular, moderately sorted, very weak silica cement, trace to common medium to dark brown argillaceous matrix, clear to opaque quartz grains, trace orange red and green grey lithics, common black coal detritus, trace coarse mica flakes, unconsolidated, very good inferred porosity, no oil fluorescence. CLAYSTONE: light to medium grey to dark brown grey, very carbonaceous in part, occasionally very finely arenaceous, very soft, very dispersive, non fissile. |