PETRO TECH PTY. LTD. A.C.N. (009 116 429) HAZELWOOD No.1 PEP 166

DAILY GEOLOGICAL REPORT No. 20 Date: 12-04-2007

Depth: 1378m Progress: 43m Days from Spud: 20

Rig: CENTURY RIG No.11 GL(AHD): 100.0m Drilling Rep: Lou DeVattimo RT: (datum) 105.2m Geologist: David Horner Last Casing: 9⅝" at 1298m

0600 hrs Update: Drill ahead with 8½" hole at 1431m. Current lithology: Claystone (90%) laminated with Sandstone (10%), background gas 0.2 units (C1 100%). Flowline temperature 44.7°C.

Comments: Drill ahead with 8½" hole to 1353m, survey 20° 88Az, POOH for BHA change, trip gas 0.3 units (C1 100%), drill ahead with 8½" hole to 1378m.

| Interval (mRT) | Hydrocarbon Show Summary | Gas |
| --- | --- | --- |
| 1335-1378 | Strzelecki Formation - No Show. | TG 0.1-0.6u C1 94% C2 6% C3+ 0 |

| Formation Tops: | Prognosed (mRT) | Actual* (mRT) | Actual* (mSS) | Difference* (High/Low) |
| --- | --- | --- | --- | --- |
| Haunted Hill Formation | 5.2 | 5.2 | +100 | 0 |
| LaTrobe Group | 10 | - | - | - |
| Strzelecki Formation | 530 | 545 | -440 | 15 Low |
| Rintouls Creek Sand (Tyers Conglomerate) | 2430 |  |  |  |
| Basement | 2730 |  |  |  |
| Total Depth | 2750 |  |  |  |

*Provisional, based on mudlog

| Lithological and Fluorescence Description |  |
| --- | --- |
| Interval (m) | Description |
| 1335-1378 | STRZELECKI FORMATION Claystone (80%) with minor laminated Sandstone (20%). CLAYSTONE: light to medium green grey to medium grey, non to very silty, occasionally very finely arenaceous with altered feldspar grains, trace very fine black carbonaceous flecks in part, common micromica, common calcite lined fractures, hard, subfissile to fissile. SANDSTONE: light to medium green grey to medium brown grey, silty to very fine, subrounded, moderately to well sorted, very strong silica and weak calcareous cements, abundant green grey siliceous matrix, common to abundant altered feldspar grains, trace black carbonaceous flecks, common calcite lined fractures, very hard, no visual porosity, no oil fluorescence. |