PETRO TECH PTY. LTD. A.C.N. (009 116 429) HAZELWOOD No.1 PEP 166

DAILY GEOLOGICAL REPORT No. 23 Date: 15-04-2007

Depth: 1603m Progress: 90m Days from Spud: 23

Rig: CENTURY RIG No.11 GL(AHD): 100.0m Drilling Rep: Lou DeVattimo RT: (datum) 105.2m Geologist: David Horner Last Casing: 9⅝" at 1298m

0600 hrs Update: Drill ahead with 8½" hole at 1623m. Current lithology Claystone (100%) with minor laminated tight very fine Sandstone (trace), background gas 0.2 units (C1 100%).

Comments: Drill ahead with 8½" hole to 1603m. Survey at 1554m = 37° at 110° Az. Carbide at 1575m = 50 units.

| Interval (mRT) | Hydrocarbon Show Summary | Gas |
| --- | --- | --- |
| 1513-1603 | Strzelecki Formation - No Show. | TG 0.1-0.5u C1 100% C2+ 0 |

| Formation Tops: | Prognosed (mRT) | Actual* (mRT) | Actual* (mSS) | Difference* (High/Low) |
| --- | --- | --- | --- | --- |
| Haunted Hill Formation | 5.2 | 5.2 | +100 | 0 |
| LaTrobe Group | 10 | - | - | - |
| Strzelecki Formation | 530 | 545 | -440 | 15 Low |
| Rintouls Creek Sand (Tyers Conglomerate) | 2430 |  |  |  |
| Basement | 2730 |  |  |  |
| Total Depth | 2750 |  |  |  |

*Provisional, based on mudlog

| Lithological and Fluorescence Description |  |
| --- | --- |
| Interval (m) | Description |
| 1513-1603 | STRZELECKI FORMATION Claystone (90%) with minor laminated Sandstone (10%). CLAYSTONE: medium to dark green grey to medium brown grey, non to very silty, occasionally very finely arenaceous with altered feldspar grains, trace very fine black carbonaceous flecks in part, common micromica, trace to rarely common calcite lined fractures, hard, subfissile to fissile. SANDSTONE: medium green grey to medium brown grey, silty to very fine, subrounded, moderately to well sorted, strong silica and weak calcareous cements, abundant green grey siliceous matrix, abundant altered feldspar grains, trace black carbonaceous flecks, trace to rarely common calcite lined fractures, hard, no visual porosity, no oil fluorescence. |