PETRO TECH PTY. LTD. A.C.N. (009 116 429) HAZELWOOD No.1 PEP 166

DAILY GEOLOGICAL REPORT No. 25 Date: 17-04-2007

Depth: 1760m Progress: 70m Days from Spud: 25

Rig: CENTURY RIG No.11 GL(AHD): 100.0m Drilling Rep: Lou DeVattimo RT: (datum) 105.2m Geologist: David Horner Last Casing: 9⅝" at 1298m

0600 hrs Update: Drill ahead with 8½" hole at 1780m.

Comments: Drill ahead with 8½" hole to 1760m. Survey at 1722m = 43° at 110° Az.

| Interval (mRT) | Hydrocarbon Show Summary | Gas |
| --- | --- | --- |
| 1690-1719 | Strzelecki Formation - No Show. | TG 0.3-1.3u C1 98% C2 2% C3+ 0 |
| 1719-1760 | Strzelecki Formation - No Show. Fracture/fault zone present around 1737m as evidenced by abundant (up to 10% of sample) of quartz and calcite vein infill material, no evidence of any open fractures or associated hydrocarbons. | TG 0.3-0.8u C1 98% C2 2% C3+ 0 |

| Formation Tops: | Prognosed (mRT) | Actual* (mRT) | Actual* (mSS) | Difference* (High/Low) |
| --- | --- | --- | --- | --- |
| Haunted Hill Formation | 5.2 | 5.2 | +100 | 0 |
| LaTrobe Group | 10 | - | - | - |
| Strzelecki Formation | 530 | 545 | -440 | 15 Low |
| Rintouls Creek Sand (Tyers Conglomerate) | 2430 |  |  |  |
| Basement | 2730 |  |  |  |
| Total Depth | 2750 |  |  |  |

*Provisional, based on mudlog

| Lithological and Fluorescence Description |  |
| --- | --- |
| Interval (m) | Description |
| 1690-1719 | STRZELECKI FORMATION Claystone (100%) grading in part to Coal (trace) and minor laminated Sandstone (trace). CLAYSTONE: medium to dark grey to medium to dark brown grey, non to very silty, rarely very finely arenaceous with altered feldspar grains, very carbonaceous in part grading to argillaceous coal, trace micromica, trace calcite lined fractures, hard, subfissile. SANDSTONE: medium green grey to medium brown grey, silty to very fine, subrounded, moderately sorted, very strong silica and weak calcareous cements, abundant green grey siliceous matrix, abundant altered feldspar grains, trace black carbonaceous flecks, trace calcite lined fractures, very hard, no visual porosity, no oil fluorescence. COAL: very dark brown to black, moderately to very argillaceous, earthy lustre, irregular fracture, hard, no fluorescence or cut. |
| 1719-1760 | STRZELECKI FORMATION Claystone (100%) with minor laminated Sandstone (trace). CLAYSTONE: medium green grey to medium to dark grey to medium brown grey, slightly to very silty, occasionally very finely arenaceous with altered feldspar grains, slightly carbonaceous in part, common micromica, common to abundant (up to 10% of sample) calcite and quartz lined fractures, hard, subfissile to fissile. SANDSTONE: medium green grey to medium brown grey, silty to very fine, subrounded, moderately sorted, strong silica and weak calcareous cements, abundant siliceous matrix, abundant altered feldspar grains, trace black carbonaceous flecks, common calcite and quartz lined fractures, hard, no visual porosity, no oil fluorescence. |