PETRO TECH PTY. LTD. A.C.N. (009 116 429) HAZELWOOD No.1 PEP 166

DAILY GEOLOGICAL REPORT No. 3 Date: 26-03-2007

Depth: 556m Progress: 73m Days from Spud: 3

Rig: CENTURY RIG No.11 GL(AHD): 100.0m Drilling Rep: Lou DeVattimo RT: (datum) 105.2m Geologist: David Horner Last Casing: 18" at 286m

0600 hrs Update: Run 13 3/8" casing.

Comments: Drill ahead with 16" hole to 523m, repair rotary drive chain, drill ahead with 16" hole to 556m, circulate hole clean, run survey, condition hole for 13 3/8" casing. NOTE: The formation encountered at 545m may either be the Golden Beach Formation or the Strzelecki Formation - no distinction is possible at the wellsite. The Volcanics encountered above this unit may either belong to the LaTrobe Group or to the Golden Beach Formation - no distinction is possible at the wellsite.

| Interval (mRT) | Hydrocarbon Show Summary | Gas |
| --- | --- | --- |
| 483-506 | LaTrobe Group - No Show. | TG trace C1 100% C2+ 0 |
| 506-545 | LaTrobe Group - No Show. | TG trace C1 100% C2+ 0 |
| 545-556 | Golden Beach Formation - No Show | TG trace C1 100% C2+ 0 |

| Formation Tops: | Prognosed (mRT) | Actual* (mRT) | Actual* (mSS) | Difference* (High/Low) |
| --- | --- | --- | --- | --- |
| Haunted Hill Formation LaTrobe Group - Yallourn Formation LaTobe Group - Morwell Formation LaTrobe Group - Traralgon Formation | 5.2 10 170 440 | 5.2 - - - | +100 - - - | 0 - - - |
| Golden Beach (Kipper Shale) | 530 | 545 | -440 | 15 Low |
| Strzelecki Formation | 1030 |  |  |  |
| Rintouls Creek Sand (Tyers Conglomerate) | 2430 |  |  |  |
| Basement | 2730 |  |  |  |
| Total Depth | 2750 |  |  |  |

*Provisional, based on mudlog

| Lithological and Fluorescence Description |  |
| --- | --- |
| Interval (m) | Description |
| 483-506 | LATROBE GROUP Sandstone (90%) with minor Claystone (10%). SANDSTONE: light brown grey, very fine to grit, dominantly very coarse, angular to subangular, moderately sorted, very weak silica cement, trace to common medium to dark brown argillaceous matrix, clear to opaque quartz grains, trace orange red and green grey lithics, common black coal detritus, trace coarse mica flakes, unconsolidated, very good inferred porosity, no oil fluorescence. CLAYSTONE: light to medium grey to dark brown grey, very carbonaceous in part, occasionally very finely arenaceous, very soft, very dispersive, non fissile. |
| 506-545 | LATROBE GROUP Volcanics weathered to Claystone (100%). CLAYSTONE: (weathered and partially weathered volcanics), off white to brick red, light green, mottled, cryptocrstalline to microcrystalline, trace very fine to very coarse yellow-red stained angular quartz grains, soft to hard, non fissile. |
| 545-556 | GOLDEN BEACH FORMATION Claystone (90%) with minor laminated Sandstone (10%). CLAYSTONE: medium grey to medium green grey, slightly silty, trace black carbonaceous flecks, trace micromica, moderately hard, subfissile. SANDSTONE: light green grey, very fine, subangular to subrounded, moderately sorted, moderate silica cement, abundant white argillaceous matrix, abundant off white altered feldspar grains, common green grey and trace red lithics, trace fine black carbonaceous detritus, hard, no visual porosity, no oil fluorescence. |