PETRO TECH PTY. LTD. A.C.N. (009 116 429) HAZELWOOD No.1 PEP 166

DAILY GEOLOGICAL REPORT No. 30 Date: 22-04-2007

Depth: 1920m Progress: 31m Days from Spud: 30

Rig: CENTURY RIG No.11 GL(AHD): 100.0m Drilling Rep: Lou DeVattimo RT: (datum) 105.2m Geologist: David Horner Last Casing: 9⅝" at 1298m

0600 hrs Update: Drill ahead with 8½" hole at 1939m.

Comments: POOH with Core No.1, lay out core barrel, RIH with new 8½" tricone drill bit, trip gas 0.7 units, drill ahead with 8½" hole to 1920m. Core No.1, cut 1886.0-1889.0 (3.0m), recovered 1886.0-1889.0m (3.0m = 100%).

| Interval (mRT) | Hydrocarbon Show Summary | Gas |
| --- | --- | --- |
| 1887-1920 | Crayfish Equivalent - No Show. | TG 0.3-0.5u C1 99% C2 1% C3+ 0 |

| Formation Tops: | Prognosed (mRT) | Actual* (mRT) | Actual* (mSS) | Difference* (High/Low) |
| --- | --- | --- | --- | --- |
| Haunted Hill Formation | 5.2 | 5.2 | +100 | 0 |
| LaTrobe Group | 10 | - | - | - |
| Strzelecki Formation | 530 | 545 | -440 | 15 Low |
| Rintouls Creek Sand (Tyers Conglomerate) | 2430 | n/p | n/p | - |
| Crayfish Equivalent | n/p | 1830 | -1725 | - |
| Basement | 2730 |  |  |  |
| Total Depth | 2750 |  |  |  |

*Provisional, based on mudlog

| Lithological and Fluorescence Description |  |
| --- | --- |
| Interval (m) | Description |
| 1887-1920 | CRAYFISH EQUIVALENT Shale (100%) with minor laminated Sandstone (trace). SHALE: very dark grey to grey black, moderately to dominantly very carbonaceous, slightly silty in part, trace vitreous black coal detritus in part, common micromica, trace calcite and quartz lined fractures, hard, subfissile to fissile, SANDSTONE: off white to medium brown grey, silty to very fine, subrounded, moderately sorted, very strong silica and weak calcareous cements, abundant white siliceous matrix, abundant altered feldspar grains, trace very fine black carbonaceous flecks, trace calcite and quartz, lined fractures, very hard, no visual porosity, no oil fluorescence. |
| CORE N0.1 1886.0-1889.0 | CORE No.1 Cut 1886.0-1889.0m (barrel jammed), Recovered 1886.0-1889.0m (3.0m = 100%). Core from 1886.0-1888.0m - left intact in aluminium sleeve. Core from 1888.0-1888.5m - sleeve cut in half, described and photographed, sleeve clamped back in place. Core from 1888.5-1889.0 - retrieved from catcher sub - badly fragmented. DESCRIPTION: 1886.0-1889.0m. SHALE: very dark grey to grey black, moderately to dominantly very carbonaceous, slightly silty in part, trace vitreous black coal detritus in part, trace to common micromica, hard, subfissile to fissile, with minor laminations of: SANDSTONE: off white to medium brown grey, silty to very fine, subrounded, moderately sorted, very strong silica and weak calcareous cements, abundant white siliceous matrix, abundant altered feldspar grains, trace very fine black carbonaceous flecks, very hard, no visual porosity, no oil fluorescence. STRUCTURE: Based on the orientation of the sandstone laminae and corrected for a well bore inclination of 40°, the sedimentary dip is either 81° from horizontal or more probably 19° from horizontal. The shale is complexly slickensided with non-mineralised surfaces, laying at variable angles from vertical to horizontal and often offset. Several small tight calcite infilled fractures are present, often with the calcite veins offset or contused by slickensided shear planes. |