PETRO TECH PTY. LTD. A.C.N. (009 116 429) HAZELWOOD No.1 PEP 166

DAILY GEOLOGICAL REPORT No. 6 Date: 29-03-2007

Depth: 556m Progress: 0m Days from Spud: 6

Rig: CENTURY RIG No.11 GL(AHD): 100.0m Drilling Rep: Lou DeVattimo RT: (datum) 105.2m Geologist: David Horner Last Casing: 13⅜" at 420m

0600 hrs Update: Finish testing BOP's, drill out cement and shoe track, perform leak off test to 150PSI, light ream to bottom.

Comments: Nipple up BOP's, pressure test BOP's, make up and RIH with 12¼" drilling assembly, tagged cement at 406m, continue testing BOP's.

| Interval (mRT) | Hydrocarbon Show Summary | Gas |
| --- | --- | --- |
|  | No new formation drilled. |  |

| Formation Tops: | Prognosed (mRT) | Actual* (mRT) | Actual* (mSS) | Difference* (High/Low) |
| --- | --- | --- | --- | --- |
| Haunted Hill Formation | 5.2 | 5.2 | +100 | 0 |
| LaTrobe Group | 10 | - | - | - |
| Strzelecki Formation | 530 | 545 | -440 | 15 Low |
| Rintouls Creek Sand (Tyers Conglomerate) | 2430 |  |  |  |
| Basement | 2730 |  |  |  |
| Total Depth | 2750 |  |  |  |

*Provisional, based on mudlog

| Lithological and Fluorescence Description |  |
| --- | --- |
| Interval (m) | Description |
|  | No new formation drilled. |