PETRO TECH PTY. LTD. A.C.N. (009 116 429) HAZELWOOD No.1 PEP 166

DAILY GEOLOGICAL REPORT No. 7 Date: 30-03-2007

Depth: 669m Progress: 113m Days from Spud: 7

Rig: CENTURY RIG No.11 GL(AHD): 100.0m Drilling Rep: Lou DeVattimo RT: (datum) 105.2m Geologist: David Horner Last Casing: 13⅜" at 420m

0600 hrs Update: Drill ahead with 12¼" hole at 699m. Current lithology: Claystone (100%) with minor laminated Sandstone (trace), no shows, background gas trace (8-18ppm C1 only).

Comments: Finish testing BOP's, drill out cement and shoe track, perform leak off test to 150PSI, light ream to bottom, drill ahead with 12¼" hole to 669m. Carbide at 579m = 21 units.

| Interval (mRT) | Hydrocarbon Show Summary | Gas |
| --- | --- | --- |
| 556-669 | Strzelecki Formation - No Show | TG 0.1-0.2u C1 100% C2+ 0 |

| Formation Tops: | Prognosed (mRT) | Actual* (mRT) | Actual* (mSS) | Difference* (High/Low) |
| --- | --- | --- | --- | --- |
| Haunted Hill Formation | 5.2 | 5.2 | +100 | 0 |
| LaTrobe Group | 10 | - | - | - |
| Strzelecki Formation | 530 | 545 | -440 | 15 Low |
| Rintouls Creek Sand (Tyers Conglomerate) | 2430 |  |  |  |
| Basement | 2730 |  |  |  |
| Total Depth | 2750 |  |  |  |

*Provisional, based on mudlog

| Lithological and Fluorescence Description |  |
| --- | --- |
| Interval (m) | Description |
| 556-669 | STRZELECKI FORMATION Claystone (90%) laminated and thinly interbedded with Sandstone (10%). CLAYSTONE: medium to dark grey, occasionally medium brown grey to medium green grey, slightly to occasionally very silty, common dispersed very fine altered feldspar grains in part, trace black carbonaceous flecks, trace micromica, trace calcite lined fractures in part, moderately hard, subfissile. SANDSTONE: light green grey, silty to very fine, subangular to subrounded, moderately sorted, strong silica cement, occasional weak calcareous cement, abundant white argillaceous matrix - matrix supported, abundant off white altered feldspar grains, common green grey and trace red lithics, trace fine black carbonaceous detritus, trace calcite lined fractures in part, hard, no visual porosity, no oil fluorescence. |