|  | ###### WELL |  | WKF W-21A |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DATE |  | 20-08-2006 |  |  |  |  |  |
|  | ###### DAY # |  | 6 |  |  |  |  |  |
| MIDNIGHT ACTIVITY |  | Drill, steer and survey 8½" production hole. |  |  | ROP DATA (LAST 24 hours) |  | m/hr | DEPTH (m) |
| MIDNIGHT DEPTH |  | 2566.0 mMDRT |  |  | MINIMUM |  | 2.6 | 2223.0 |
| TVD |  | 2403.9 mTVDRT |  |  | MAXIMUM |  | 97.5 | 2424.0 |
| PROGRESS LAST 24 hrs |  | 365.0 m |  |  | AVERAGE |  | 27.4 | 2201.0-2566.0 |

-

|  | TOP (mMDRT) | TOP (mTVDRT) | TOP (mTVDSS) RTtoMSL (33.4m) | HIGH/ LOW (mTVD) | PROGNOSED TOP (mMDRT) | PROGNOSED TOP (mTVDRT) |
| --- | --- | --- | --- | --- | --- | --- |
| High Velocity Channel | 1533.0 | 1411.7 | -1378.3 | 18.3 LOW | 1513.9 | 1393.4 |
| Lakes Entrance | 2178.5 | 2030.4 | -1997.0 | 3.0 HIGH | 2176.9 | 2033.4 |
| Top of Latrobe | 2417.0 | 2259.7 | -2226.3 | 0.3 HIGH | 2413.9 | 2259.4 |
| #### P1.1 | 2436.5 | 2278.6 | -2245.2 | 0.7 HIGH | 2434.8 | 2279.4 |
| M1.2L (SBP3) | 2450.0 | 2291.6 | -2258.2 | 0.8 HIGH | 2448.4 | 2292.4 |
| Top of M1.3U (PSB7) | 2458.0 | 2299.4 | -2266.0 | 2.0 HIGH | 2457.7 | 2301.4 |
| PS4 Sand (PSB5) | 2465.0 | 2306.1 | -2272.7 | 0.3 HIGH | 2462.9 | 2306.4 |
| Top M1.3 L (PSB4) | 2478.0 | 2318.7 | -2285.3 | 0.3 LOW | 2475.5 | 2318.4 |
| Base M1.3L / M1.4U | 2500.0 | 2340.0 | -2306.6 | 1.6 LOW | 2496.4 | 2338.4 |
| M14 Coal / Top M1.4L | 2505.0 | 2344.8 | -2311.4 | 0.8 HIGH | 2503.0 | 2345.4 |
| Top M1.5 | 2514.0 | 2353.5 | -2320.1 | 2.9 HIGH | 2515.0 | 2356.4 |
| Top M1.7 | 2527.0 | 2366.1 | -2332.7 | 3.3 HIGH | 2529.0 | 2369.4 |
| Total Depth | 2575.0 | 2412.6 | -2379.2 |  | 2564.0 | 2403.4 |

| ##### Current Formation | Latrobe Group: Sandstone / Siltstone. |
| --- | --- |

| GAS DATA | DEPTH (m) | TOTAL GAS (U) | C1 % | C2 % | ## C3 % | iC4 % | nC4 % | C5 % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MAX (FCP/FGP) | 2497.0 | 85.0 | 0.226 | 0.060 | 0.105 | 0.054 | 0.095 | 0.091 |
| MAX (Reserval) | 2476.0 | 418.0 | 1.337 | 0.424 | 0.656 | 0.249 | 0.391 | 0.328 |
| BG (FCP/FGP) | 2201.0-2417.0 2417.0-2566.0 | 5.0 40.0 | 0.092 0.143 | 0.003 0.036 | 0.049 | 0.023 | 0.038 | 0.035 |
| BG (Reserval) | 2201.0-2417.0 2417.0-2566.0 | 15.0 200.0 | 0.223 0.390 | 0.014 0.153 | 0.013 0.295 | 0.003 0.130 | 0.002 0.203 | 0.184 |
| TRIP GAS (FCP) |  |  |  |  |  |  |  |  |
| TRIP GAS Reserval) |  |  |  |  |  |  |  |  |

| BIT DATA | PRESENT BIT | LAST BIT |
| --- | --- | --- |
| BIT No. | 1RR1 |  |
| SIZE (in) | 8½ |  |
| MANUFACTURER/TYPE | Smith S616PX |  |
| JETS (/32 inch) | 6 x 20 |  |
| DEPTH IN (m) | 667.0 |  |
| DEPTH OUT (m) | IN HOLE |  |
| METRES ON RUN (m) | 1899.0 |  |
| HRS ON BTM 24hrs/total | 13.3 / 68.9 |  |
| BIT CONDITION |  |  |
| RPM RANGE LAST 24 hrs | TDRPM: 105-120 / MMRPM: 176-179 |  |
| Pump Hrs 24 hrs / total | 20.2 / 110.2 |  |
| Krevs 24 hrs / total | 230.0 / 1056.0 |  |
| 24 HOUR SUMMARY | Drill, steer and survey 8½" production hole from 2201.0 mMDRT to 2502.0 mMDRT. Observe pressure loss of 500 psi over 3 hours. Flow check and pull out of hole from 2502.0 mMDRT to 2104.0 mMDRT. Lay out washed joint of drill pipe and make up replacement joint. Circulate and work string while observing stand pipe and MWD pressures. Run in hole and wash last 2 stands to bottom. Drill and survey 8½" production hole from 2502.0 mMDRT to 2566.0 mMDRT. |  |
| 05:00 AM UPDATE 21-08-2006. ###### DEPTH AT 05:00 hours | Drill and survey 8½" production hole from 2566.0 mMDRT to 2575.0 mMDRT. Rotate, work string and circulate hole clean at Total Depth. Pump 30 bbl super sweep. Circulate and work string while racking back 1 stand per ½ hour from 2575.0 mMDRT to 2507.0 mMDRT. Flow check, slug pipe and pull out of hole from 2507.0 mMDRT to 1646.0 mMDRT. 2575.0 mMDRT (2412.6 mTVDRT) |  |