| Date: | 20 Oct 2009 | Report Number: | 4 |
| --- | --- | --- | --- |
| Report Period: | 0000-2400 Hours | Last Casing Shoe: | 569.44 mMDRT |
| Depth @ 2400hrs: | 861 mMDRT | Last Casing Size: | 762 (30”) |
| Previous 2400hr depth: | 572.5 mMDRT | Current Bit Size: | 444 mm (17”) |
| Progress: | 288.5 m | FIT: | N/A |
| Lag Depth: | Returns to seafloor | Mud Weight: | 1.04 sg |
| Water Depth: | 503.0 m LAT | ECD: | N/A |
| RT: | 21.5 m | Mud Type: | Hi-Vis PHG/Guar Gum |
| Last Survey Depth: | 799.04 mMDRT | Mud Chlorides: | 300 |
| Deviation: | Inc 0.53° Azi 124.88° | Est. Pore Press: | 1.03 sg EMW |

## OPERATIONS SUMMARY

| ### 24 HOUR SUMMARY TO MIDNIGHT: | Picked up PGB and confirmed engagement with conductor. Ran and landed 762 mm (30”) conductor and set 508 mm (20”) shoe at 569.44 mMDRT. Cemented the conductor, released the running tool and POOH. Laid down 914 mm (36”) BHA and cement head. RIH with 444 mm (17-1/2”) BHA and tagged top of cement at 567.25 mMDRT. Drilled out shoe track. Drilled 444 mm (17.5” hole from 572.5 to 861 mMDRT. |
| --- | --- |
| ### NEXT 24 HOURS: | Drill 444 mm (17-1/2”) hole to section TD. Circulate hole clean prior to pumping displacement fluid and POOH. Slip and cut drill line. Prepare to run 340 mm (13-3/8”) casing. |
| ### MIDNIGHT TO 0600hrs SUMMARY: | Drilled 444 mm (17.5”) hole from 861 to 1211 mMDRT |
| ### CURRENT OPERATION @ 06:00hrs: | - Drilling 444 mm (17.5”) hole at 1211 mMDRT |

## GEOLOGICAL SUMMARY

LITHOLOGY

- Returns to seafloor.

HYDROCARBON FLUORESCENCE

Returns to seafloor.

- GAS SUMMARY

| Background | - Total GAS (%) | - C1 - (%) | - C2 - (%) | - C3 - (%) | - IC4 - (%) | - NC4 - (%) | - IC5 - (%) | - NC5 - (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No returns | - | - | - | - | - | - | - | - |
| - Peaks | - Total GAS (%) | - C1 - (%) | - C2 - (%) | - C3 - (%) | - iC4 - (%) | - NC4 - (%) | - IC5 - (%) | - NC5 - (%) |
| - N/A | - | - | - | - | - | - | - | - |
| Connection | - Total GAS (%) | - C1 - (%) | - C2 - (%) | - C3 - (%) | - IC4 - (%) | - NC4 - (%) | - IC5 - (%) | - NC5 - (%) |
| - N/A | - | - | - | - | - | - | - | - |
| Trip | - Total GAS (%) | - C1 - (%) | - C2 - (%) | - C3 - (%) | - IC4 - (%) | - NC4 - (%) | - IC5 - (%) | - NC5 - (%) |
| - N/A | - | - | - | - | - | - | - | - |

FORMATION PRESSURE ESTIMATION

| - ITEM | - REMARKS |
| --- | --- |
| - Mud weight | - 1.04 sg |
| - Connection gas | - N/A |
| - Trip gas | - N/A |
| - Background gas | - N/A |
| - Dxc exponent | - No significant cut backs. |
| - ROP | - Normal |
| - Ditch cuttings | - N/A |
| - Mud temperature out | - N/A |
| - MWD resistivity | - N/A |
| - Pore Pressure Estimate | - 1.03 sg EMW (estimate from town) |

- SAMPLE QUALITY

No returns.

### MUDLOGGING EQUIPMENT/PERSONNEL

Personnel: 2x Data Engineers, 4x Mudloggers (sharing sample catching duties).

Equipment: All consumables checked and sufficient to complete the well.

### MWD/LWD

Personnel:  2x LWD Engineers

2x Directional Drillers

Equipment: Primary tool for the 444 mm (17-1/2”) is in the hole.  Back-up checked and on deck. Tools for the 311 mm (12-1/4”) section have not yet arrived to the rig.

###### WIRELINE

Personnel: No crew onboard.

REMARKS

None

### WELLSITE GEOLOGISTS

David Hartney / Todd Teasdale

| FIELD REPORTED PROVISIONAL FORMATION TOPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Formation | Marker | Predicted Top (m) |  |  | Actual Top (m) |  |  | Diff. | Reason |
|  |  | MDRT | TVDSS | +/- | MDRT | TVDRT | TVDSS | (MD) |  |
| Seabed |  | 506.5 | 485 | 15 | 524.5 | 524.5 | 503.0 | +18 | Incorrect velocity in seismic model (+18m applied) |
| Port Campbell Limestone |  | 749.5 | 728.0 | 30 | NP | NP | NP |  |  |
| Gellibrand Marl |  | 1040.5 | 1019.0 | 30 | 1060 | 1060 | 1038.5 | 19.5 L | ROP decrease |
| Base Miocene Marker |  | 1331.5 | 1310.0 | 40 |  |  |  |  |  |
| Narrawaturk Marl |  | 1603.5 | 1582.0 | 70 |  |  |  |  |  |
| Dilwyn Formation |  | 1658.5 | 1637.0 | 70 |  |  |  |  |  |
| Massacre Formation |  | 1708.5 | 1687.0 | 70 |  |  |  |  |  |
| Skull Creek Formation |  | 2105.5 | 2084.0 | 90 |  |  |  |  |  |
| Thylacine Member |  | 2832.5 | 2811.0 | 170 |  |  |  |  |  |
| Flaxman Formation |  | 2858.5 | 2837.0 | 180 |  |  |  |  |  |
| Waare Formation |  | 3154.5 | 3133.0 | 220 |  |  |  |  |  |
| Eumeralla Formation |  | 3276.5 | 3255.0 | 240 |  |  |  |  |  |
| TD |  | 3326.0 | 3304.4 | 240 |  |  |  |  |  |
| TD max |  | 3566.0 | 3544.5 |  |  |  |  |  |  |

Note: All depths LAT

- DEPTH TABLE

|  | mMDRT unless otherwise stated | Precision of depth quoted | Remark |
| --- | --- | --- | --- |
| RTE | 21.5 above LAT | 1dp |  |
| Mudline | 524.5 | 1dp |  |
| Water Depth | 524.5 | 1dp |  |
| 914 mm (36”) TD | 572.5 | 1dp |  |
| 508 mm (20”) csg shoe | 569.44 | 2dp | Tagged TOC @ 567.25 |
| 445mm (17-1/2”) TD |  | 0dp |  |
| 340mm (13-3/8”) csg shoe |  | 0dp |  |
| 311mm (12-1/4”) TD |  | 0dp |  |

CUTTINGS SAMPLING SUMMARY

| Depth Interval (mMDRT) | Sampling rate | Samples missed |
| --- | --- | --- |
| No returns |  |  |

MUD / MUD FILTRATE SAMPLES

| Depth (mMDRT) | Sample type |
| --- | --- |
| None |  |

ISOTUBE SAMPLES

| Depth (mMDRT) | Date | Time | TG% | Remarks |
| --- | --- | --- | --- | --- |
| None |  |  |  |  |