| Date: | 22 Oct 2009 | Report Number: | 5 |
| --- | --- | --- | --- |
| Report Period: | 0000-2400 Hours | Last Casing Shoe: | 1278.6 mMDRT |
| Depth @ 2400hrs: | 1284 mMDRT | Last Casing Size: | 340 mm (13-3/8”) |
| Previous 2400hr depth: | 1284 mMDRT | Current Bit Size: | N/A |
| Progress: | 0 m | FIT: | N/A |
| Lag Depth: | Returns to seafloor | Mud Weight: | 1.03 sg |
| Water Depth: | 503.0 m LAT | ECD: | N/A |
| RT: | 21.5 m | Mud Type: | Hi-Vis PHG/Guar Gum |
| Last Survey Depth: | 1251.88 mMDRT | Mud Chlorides: | 350 |
| Deviation: | Inc 0.96° Azi 60.07° | Est. Pore Press: | 1.03 sg EMW |

## OPERATIONS SUMMARY

| ### 24 HOUR SUMMARY TO MIDNIGHT: | Made up and ran 476 mm (18-3/4”) well head running tool to well head. Ran in hole and landed 340 mm (13-3/8”) casing. Completed pressure testing of surface lines and cemented casing. Released running tool and POOH and laid out. Held JHA prior to picking up BOP. Installed guide lines and beacon. Continued to run in hole BOPs on riser. |
| --- | --- |
| ### NEXT 24 HOURS: | Land the BOPs. Test 476 mm (18.75”) well head connector. Make up and run 311 mm (12.25”) BHA. Drill out shoe track and displace the well to 1.30 sg Ultradrill mud. |
| ### MIDNIGHT TO 0600hrs SUMMARY: | Ran BOPs on riser. |
| ### CURRENT OPERATION @ 06:00hrs: | - Running BOPs on riser. |

## GEOLOGICAL SUMMARY

LITHOLOGY

- Returns to seafloor.

HYDROCARBON FLUORESCENCE

Returns to seafloor.

- GAS SUMMARY

| Background | - Total GAS (%) | - C1 - (%) | - C2 - (%) | - C3 - (%) | - IC4 - (%) | - NC4 - (%) | - IC5 - (%) | - NC5 - (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No returns | - | - | - | - | - | - | - | - |
| - Peaks | - Total GAS (%) | - C1 - (%) | - C2 - (%) | - C3 - (%) | - iC4 - (%) | - NC4 - (%) | - IC5 - (%) | - NC5 - (%) |
| - N/A | - | - | - | - | - | - | - | - |
| Connection | - Total GAS (%) | - C1 - (%) | - C2 - (%) | - C3 - (%) | - IC4 - (%) | - NC4 - (%) | - IC5 - (%) | - NC5 - (%) |
| - N/A | - | - | - | - | - | - | - | - |
| Trip | - Total GAS (%) | - C1 - (%) | - C2 - (%) | - C3 - (%) | - IC4 - (%) | - NC4 - (%) | - IC5 - (%) | - NC5 - (%) |
| - N/A | - | - | - | - | - | - | - | - |

FORMATION PRESSURE ESTIMATION

| - ITEM | - REMARKS |
| --- | --- |
| - Mud weight | - 1.03 sg |
| - Connection gas | - N/A |
| - Trip gas | - N/A |
| - Background gas | - N/A |
| - Dxc exponent | - Minor cutback at 1104 mMDRT. No other cut backs observed. |
| - ROP | - Normal reflecting changes in formation |
| - Ditch cuttings | - N/A |
| - Mud temperature out | - N/A |
| - MWD resistivity | - N/A |
| - Pore Pressure Estimate | - 1.03 sg EMW (estimate from town) |

- SAMPLE QUALITY

No returns.

### MUDLOGGING EQUIPMENT/PERSONNEL

Personnel: 2 x Data Engineers, 4 x Mudloggers (sharing sample catching duties).

Equipment: All consumables checked and sufficient to complete the well.

### MWD/LWD

Personnel:  3 x LWD Engineers.

2 x Directional Drillers.

Equipment: The primary and back-up 311 mm (12-1/4”) tools are on deck. Due to running riser operations they have not been able to start checking the tools. The 444 mm (17.5”) tools are being prepared for back load.

###### WIRELINE

Personnel: No crew onboard.

REMARKS

2 x Petrotec specialists onboard to supervise addition of mud tracer to system during 311 mm (12.25”) hole section.

### WELLSITE GEOLOGISTS

David Hartney / Todd Teasdale

| FIELD REPORTED PROVISIONAL FORMATION TOPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Formation | Marker | Predicted Top (m) |  |  | Actual Top (m) |  |  | Diff. | Reason |
|  |  | MDRT | TVDSS | +/- | MDRT | TVDRT | TVDSS | (MD) |  |
| Seabed |  | 506.5 | 485 | 15 | 524.5 | 524.5 | 503.0 | +18 | Incorrect velocity in seismic model (+18m applied) |
| Port Campbell Limestone |  | 749.5 | 728.0 | 30 |  |  |  |  |  |
| Gellibrand Marl |  | 1040.5 | 1019.0 | 30 | 1060 | 1059.95 | 1038.45 | +19.45 | Decrease in ROP |
| Base Miocene Marker |  | 1331.5 | 1310.0 | 40 |  |  |  |  |  |
| Narrawaturk Marl |  | 1603.5 | 1582.0 | 70 |  |  |  |  |  |
| Dilwyn Formation |  | 1658.5 | 1637.0 | 70 |  |  |  |  |  |
| Massacre Formation |  | 1708.5 | 1687.0 | 70 |  |  |  |  |  |
| Skull Creek Formation |  | 2105.5 | 2084.0 | 90 |  |  |  |  |  |
| Thylacine Member |  | 2832.5 | 2811.0 | 170 |  |  |  |  |  |
| Flaxman Formation |  | 2858.5 | 2837.0 | 180 |  |  |  |  |  |
| Waare Formation |  | 3154.5 | 3133.0 | 220 |  |  |  |  |  |
| Eumeralla Formation |  | 3276.5 | 3255.0 | 240 |  |  |  |  |  |
| TD |  | 3326.0 | 3304.4 | 240 |  |  |  |  |  |
| TD max |  | 3566.0 | 3544.5 |  |  |  |  |  |  |

Note: All depths LAT

- DEPTH TABLE

|  | mMDRT unless otherwise stated | Precision of depth quoted | Remark |
| --- | --- | --- | --- |
| RTE | 21.5 above LAT | 1dp |  |
| Mudline | 524.5 | 1dp |  |
| Water Depth | 524.5 | 1dp |  |
| 914 mm (36”) TD | 572.5 | 1dp |  |
| 508 mm (20”) csg shoe | 569.44 | 2dp | Tagged TOC @ 567.25 |
| 445mm (17-1/2”) TD | 1284.0 | 1dp |  |
| 340mm (13-3/8”) csg shoe | 1278.6 | 1dp |  |
| 311mm (12-1/4”) TD |  | 0dp |  |

CUTTINGS SAMPLING SUMMARY

| Depth Interval (mMDRT) | Sampling rate | Samples missed |
| --- | --- | --- |
| No returns |  |  |

MUD / MUD FILTRATE SAMPLES

| Depth (mMDRT) | Sample type |
| --- | --- |
| None |  |

ISOTUBE SAMPLES

| Depth (mMDRT) | Date | Time | TG% | Remarks |
| --- | --- | --- | --- | --- |
| None |  |  |  |  |